工作手册

中国民用航空局飞行标准司

_______________________________________________________________________________

编  号： WM-FS-2015-XX

下发日期：2015年X月X日

外航停机坪检查单和工作指南

（征求意见稿）

_______________________________________________________________________________

目录

1.目的	3

2.适用范围	3

3.参考资料	3

4.停机坪检查概述	3

4.1 检查目的	3

4.2 检查计划及执行	3

4.3监察员资格要求	4

4.4检查所需的设备和证件	4

4.5检查任务的分工	4

4.6检查的准备	4

4.7外航人员的参与	5

4.8检查一般准则	5

4.9停机坪检查单概述	6

4.10停机坪检查单填写说明	6

5.停机坪检查项目指南	7

6．根据检查结果采取的行动	7

6.1证据的保存	7

6.2问题的依据	7

6.3问题分类	7

6.4问题项目的处理	8

7.后续措施	9

7.1 数据收集	9

7.2 分析评估	9

7.3 后续行动	9

7.4 《发现问题通知书》的填写和处理	10

8.外航检查参考用语	10

8.1检查用语样例	10

8.2外航监察沟通障碍的应对	10

附件A  外航停机坪检查单	12

附件B  检查项目指南	19

附件C  预设问题和严重性分类	32

附件D  发现问题通知书	56

附件E  通知外国民航局模板	57

附件F  外航检查用语样例	58

外航停机坪检查和工作指南

# 1.目的

为进一步规范局方对外航停机坪检查工作，为局方监察员工作提供指导，制定本工作手册。

本工作手册替代中国民航局飞行标准司于2007年公布的《外国公共航空运输承运人审定及监督程序手册》（以下简称外航监察员手册）的第三章“持续监督检查”的第二节“停机坪监察”的内容和2008年8月颁布的《CCAR-129部停机坪检查工作单》。

# 2.适用范围

本工作手册适用于中国民航飞行标准监察员对外国公共航空运输承运人航空器(以下简称外航运营人)的停机坪检查工作。

# 3.参考资料

a.《国际民用航空公约》及其附件

b.《运行检查、核证和持续监督程序手册》（ICAO Doc8335）

c.《外国公共航空运输承运人运行合格审定规则》（CCAR-129）

d.《CCAR-129部运行规范标准格式》（AC-129-001）

e.《外国航空承运人飞机在中国境内的维修》（AC-129-002）

f.《外国公共航空运输承运人审定及监督程序手册》

g. EASA《SAFA Ramp Inspections Guidance material Version 2.0》

h. FAA Order 8900.1《Flight Standards Information Management System Volume 12》

# 4.停机坪检查概述

## 4.1 检查目的

监察员通过在地面对机组及飞机的检查可以有效评估外航运营人的运行情况，包括对机组人员的准备情况、过站程序、飞行后程序、机组人员和地面工作人员采用的常规方法及程序的遵守情况。通过以上评估来判断该外航运营人是否满足国际民用航空公约及附件、中国民航规章和安全运行的相关要求。

## 4.2 检查计划及执行

a.民航各地区管理局及派出机构应当依据中国民航局飞行标准司每年度制定的《安全运行监察大纲》制定次年的外航停机坪检查计划，一般应在每年12月底前完成。

b.民航各地区管理局及派出机构应当尽可能合理分配全年工作计划，一般应当在每年11月底前完成当年检查任务。

c.民航各地区管理局及派出机构应当使用飞行标准监督管理系统（FSOP）进行检查任务布置、数据收集和分析评估工作。

d.外航停机坪检查单分成飞行运行和适航维修两个部分，以上两个部分可以分别在FSOP系统进行任务布置、数据收集和分析评估。

e．临时增加的检查、外航专项检查和民航安全大检查等计划外停机坪检查工作也可以使用FSOP系统完成检查任务布置、数据收集和分析评估工作。

## 4.3监察员资格要求

监察员应当持有中国民航局颁发的监察员证件，在实施停机坪检查之前应当接受培训，熟悉检查单、检查程序并能够使用英语进行沟通。

## 4.4检查所需的设备和证件

监察员应当携带监察员证、机场通行证件和必要的设备，如手电筒、数码相机、手机、荧光背心、护耳用具等适合当时环境条件的装备。

## 4.5检查任务的分工

如有两名（含）以上监察员实施停机坪检查时，为确保充分、高效进行检查，监察员可以分工完成对驾驶舱、客舱、航空器外部的绕机检查和货舱的检查。

## 4.6检查的准备

实施停机坪检查前，做好以下准备工作，包括：

a.下载并打印相关检查单；

b.准备证件和设备工具等。

c.查询航班相关信息。与运营人的代理人员或排班人员进行协调（或联系机场、空管部门获取航班相关信息），选择检查的航班并确定所需设备及检查时间，并查询所选航班的机场、气象、情报等相关信息。

d.由于航空器过站时间有限，可提前在检查单中填写航班信息，如公司名称、航班号和机型等。

e.做好人员分工。

f.按检查单要求，提前列出重点检查项目并规划最佳检查方案。

g.实施停机坪检查前可以登录FSOP系统（fsop.caac.gov.cn），了解该外航运营人的运行规范批准情况，以往检查中发现的问题等（如有）。

h.可以登录飞行安全信息网(safety.caac.gov.cn)，了解该外航运营人在我国境内运行的不安全事件信息。

i.预习检查单内容，包括“检查指南”等本工作手册相关内容。

j.如果监察员需要进一步了解外航运营人的公司手册或程序，可以与所属地区管理局外航处或运行规范管理局外航处进行协调。

## 4.7外航人员的参与

机长、机长代表、客舱或相关外航工作人员应该在场。如必要，可以联系外航运营人在机场办事处的代表陪同监察员进行检查。外航运营人代表可提供有效帮助，促进监察员与机组人员或外航运营人所在国基地的沟通。

## 4.8检查一般准则

4.8.1限定时间。监察员可以在不提前通知外航运营人的情况下进行停机坪检查。监察员应当采取一定策略与方法，避免干扰乘客登机离机或妨碍地面服务，避免与乘客进行不必要的接触。如条件允许，可以分别在机内和机外同时执行检查任务。

4.8.2检查项目。监察员应尽量完成检查单中的所有项目。若条件（时间或人为因素等）不允许，可能一次不能观察所有相关的活动。因此，监察员应该在检查时有所侧重，但应当至少完成检查单中带※号(已添加底纹背景)的项目。其他检查项目可以在下一次检查中实施。

4.8.3检查重点。监察员应当根据检查前所做的准备、以往检查记录和工作经验来重点检查对安全有关键影响的要素。这些要素的确定取决于待查航班的特点，应考虑的要素包括：

a.不同检查项目、检查重点之间的重要性不同。比如运行飞行计划（OFP）造成的安全影响要高于驾驶舱的清洁度、整齐度和总的情况，应优先考虑。

b.航空器构型和类型的不同。对于载货航空器和载客航空器来说，检查重点不同。

c.对待查的外航运营人在之前检查中发现问题的复查和确认。

d.机型与机龄。航空器的机龄会对安全产生重要的影响。

4.8.4监察员与外航人员的配合。检查可能会对机组成员、地面货运代理人及其他地勤人员的工作造成一定不便，监察员应当尽全力将不便降到最低程度，比如：

a.登机时首先向机长介绍检查组成员并说明来意；

b.向飞行机组成员索要航空器文件时应当力求表述精确完整，以便占用最少的谈话时间，留给机组人员充足时间进行飞行前准备；

c.应要求乘务长指派一名客舱机组人员协助完成客舱检查任务；

d.检查任务完成后，应就检查情况与机长进行讲评和交流；

e.应告知货物装载人员在执行货舱检查中可能给他们造成妨碍；

f.在驾驶舱执行检查时，原则上给予直接参与飞行准备的人员工作优先权。

## 4.9停机坪检查单概述

飞行运行检查和适航维修两个部分检查单包含的总检查项目为50项，其中有3个检查项目（A部分中的第7项，第12项，第20项是重合的。飞行运行检查的部分主要包括对飞机驾驶舱和客舱中安装的设备，机组人员资质和运行文件进行检查，检查项目共32项（其中A部分20项，B部分12项）；适航维修检查部分主要包括对飞机适航状态和飞机外部情况进行检查，检查项目共18项（其中A部分3项，C部分13项，D部分2项）。

在进行停机坪检查时，监察员应当使用相应的检查单实施检查。该检查单用于提示监察员进行检查时可观察和评估的事项。在停机坪检查中的某些事项和问题可能没有包含在检查单中。当遇到此种情况，监察员应当依靠个人专业知识和经验，确定适当的检查范围。

检查单是基于相关规章或政策要求而制定的。检查单通过对问题进行细化、分解描述和补充，使回答问题所需进行的观察活动具体化。其作用类似于监察员手册,是监察员在检查中参考使用的工作指南。监察员在使用检查单过程中，针对相应问题，不必对其中的细化项逐一检查作答，而是根据检查单选择适用项目实施抽查。适用项目检查结果为合格表明，监察员对检查项目的抽查中没有发现问题，并不代表对检查单中所有适用项均已实施检查且结果为合格。

另外，由于检查的效果来源于发现的危险源和存在的风险，检查单中的细化项为监察员指出了相关检查项目的风险点所在，监察员在时间允许情况下应尽量完成更多项目的观察。针对发现的问题，监察员必须立即向相关人员提出，让外航运营人有机会采取纠正措施。

## 4.10停机坪检查单填写说明

本手册附件A中为《外航停机坪检查单》模板。每个停机坪检查单分为飞行运行和适航维修两个部分。每个部分包含的具体信息为：

a.基本信息：需要监察员用英文填写。

b.检查项目：在每项中可以看到“focus on”列对应的检查重点，也可以参考本手册附件B对应的检查项目指南。每次停机坪检查必检项目已标出※号，并有灰色底纹。

在（check）栏中填写检查结果，填写S(satisfied)表示合格，填写U(unsatisfied)，表示不合格，填写N表示未观察（not checked）。

如果※号必检项目没有完成，不视为完成一次停机坪检查。

c.发现问题：如果在第二部分某一检查项目填写U，则需要在第三部分发现问题（findings），填写具体问题项目序号，以及对应的检查重点。如A-01-a，问题描述需要用中英文同时填写，描述具体内容可参考本手册第6章节“根据检查结果采取的行动”。如果发现问题不在对应的检查重点之内，则直接写出问题项目序号，如A-01。

d.备注：可以用中文或英文填写。主要填写不能当场作出问题判断而需要后续进一步核实的项目，以及其他必要的备注说明事项等。

e.签字:监察员签字应使用中文及拼音。如只有1名监察员参与检查，监察员签名栏2处的表格留空。

# 5.停机坪检查项目指南

本手册附件B部分为具体检查项目的评估标准，作为监察员工作的指导。

# 6．根据检查结果采取的行动

## 6.1证据的保存

如果检查确认发现问题，监察员应尽可能记录和保存发现的违规情况。例如将出现故障的地方拍照，也可拍下用于评估技术故障的制造商参考资料、航空器飞行记录本或复印件。这些证据可以用来解释和详细说明检查发现问题，也可在检查发现问题受到质疑时用来作为文件记录的佐证。

## 6.2问题的依据

如果检查发现违反标准的情况，监察员必须确认适用的法律法规规章及相应条款，避免使用所谓的“经验做法”。

## 6.3问题分类

检查发现问题的等级取决于所发现问题对应安全的影响程度。在确定发现问题的等级之前，应当对现场情况进行合理的评估。根据国际民航组织DOC8335文件要求，监察员对外航运营人检查中发现的问题分为轻微（1类），严重（2类）和重大（3类）三个等级。针对不同等级问题对应采取的后续行动是不同的。本手册附件C中给出了不同检查项目的预设问题、问题严重程度分类，附件B中也有相应检查项目的工作指南。但这些标准仅供参考。尽管该列表很详细，但也不可能覆盖到所有可能出现的情况。在记录发现问题时，监察员应当优先使用此预设问题。对于那些预设问题列表中没有列出的检查发现问题，监察员应当根据发现问题和经验作出正确判断并将其划入合适的等级。如在停机坪检查中的某些问题事项没有包含在检查单及问题等级中，监察员应当依靠个人专业知识和经验，确定适当的问题和分类。

| 检查结果 的严重性 | 行动 |  |  |
| --- | --- | --- | --- |
|  | 通知机长 | 通知外航运营人采取纠正措施 | 通知运营人所在国民航局或航空器登记国民航局 |
| 轻微（1类） | 是 | 否 | 否 |
| 严重（2类） | 是 | 是 | 否 |
| 重大（3类） | 是 | 是 | 是 |

监察员应当对检查遇到的情况做好合理评估，确认违规情况与有关规定之间的差别，对检查结果进行正确归类。监察员不应当在没有进行深入调查的情况下而把发现问题评估为3类检查结果。比如，一些情况下，航空器合格证或机组人员执照的缺失可能会对安全造成重大影响。然而，在得到进一步证据后，可能对安全的影响比预见的要小。一个典型的例子是一名飞行机组成员的执照不在航空器上，如果在起飞前有证据表明该机组成员的确持有有效执照，而只是没把执照带在身边，则对安全的影响很小，那么应该评估为“机组成员拥有合适的执照但在检查时并未携带”的1类检查结果。如果起飞前没有得到确认，则可评估为3类检查结果“机组成员没有合适的执照”，并要求该航空器被授权起飞前采取纠正措施。在收到相关执照的确认信息之前，飞行机组成员不得执行飞行任务。

上述检查方法也适用于航空器文件（注册证书、适航证、无线电台执照）及航空公司授权文件的检查（航空运营人合格证、运行规范），这些证书的问题不一定被评估为重大级别的3类检查结果，（例如注册执照、无线电台执照）。

## 6.4问题项目的处理

6.4.1当场纠正。对不符合要求的项目，监察员必须立即告知机长和相关人员，使外航运营人有机会在不影响航班正常运行的情况，立即采取纠正措施。监察员必须查证所采取的纠正措施符合国际民用航空公约及附件、中国民航规章和相关手册的要求。

6.4.2事后通知。对不符合要求的项目，如果外航运营人不能够当场采取纠正措施，监察员应当以《发现问题通知书》的形式通知外航运营人，存在重大问题时还应当通知运营人所在国民航局或航空器登记国民航局。

6.4.3进一步核实。对于不能当场作出判断而需要后续进一步核实的项目，监察员应当在检查单备注项填写需要后续核实的情况。

6.4.4不在检查单内的问题。对明显不符合国际标准，但对飞行安全和运行标准不构成直接的影响的问题，应当在检查单备注项记录为其他类问题，说明视情况通知运营人并作FSOP数据收集。

6.4.5航班延误。监察员一般应当避免外航运营人航空器起飞延误。但是，当监察员发现可能存在对飞行安全有重大影响的问题或要求进一步调查以核实问题时，可以允许延误，比如：

a.轮胎磨损超过限度（无法看见中间凹槽），但必须参考适用的《航空器维修手册》以确定实际的磨损程度是否超标；

b.必须根据适用的《航空器维修手册》核查燃油泄漏（例如每分钟5滴），确定燃油实际泄漏程度是否超标；

c.飞行机组成员未持有有效执照。监察员必须从外航运营人总部得到核实，比如可提供执照复印件供监察员核实，以确认该飞行机组成员持有有效执照。

6.4.6限制航空器起飞。监察员应当尽可能避免扣留外航运营人航空器。但是，当监察员发现可能存在对飞行安全有重大影响的问题时，可以扣留外航运营人航空器。在此情况下，监察员必须事先告知所属地区管理局，由所属地区管理局通知运行规范管理局，由运行规范管理局协商外航运营人总部、外航运营人所在国民航局后方可实施。

如果出现影响适航性的航空器的损坏，由运行规范管理局应当与航空器登记国民航局直接沟通。根据附件8 的规定，该民航局决定外航运营人航空器恢复飞行的条件。

# 7.后续措施

## 7.1 数据收集

监察员在FSOP系统中进行数据收集。

## 7.2 分析评估

7.2.1负责数据收集的监察员实施分析评估。

7.2.2填写要点

a.发现问题通知单应当由进行检查的监察员在FSOP系统中使用中英文填写初稿。

b.检查项目序号应当写出检查单对应的检查项目编号，如A-01-a,问题描述可以参考本手册附件C中的预设问题填写，“参考材料”应当参考本手册附件B中“参考文件”写出对应的国际民航组织公约及附件,中国民航规章及相应的运行要求。

c.问题分类参考本手册附件C中的预设问题的分类进行填写。对于轻微Minor类问题，如外航已当场完成整改，一般无需通知外航运营人，如当场无法完成整改，则应当写入《发现问题通知书》；严重类significant和重大类Major问题应当写入《发现问题通知书》。

## 7.3 后续行动

根据检查结果确定后续措施。措施包括：1）增加1次停机坪检查，2）建议所在地区管理局采取后续行动，向外航发出《发现问题通知单》， 3）其他措施，包括建议所在地区管理局进行行政约见、进行行政处罚、通知外航运营人所在国民航局，建议运行规范管理局按照《外国公共运输航空承运人运行评估管理程序》进行扣分等。

## 7.4 《发现问题通知书》的填写和处理

7.4.1 填写要求

a.整改要求视情况填写，如需要公司提交哪些材料等，由监察员自主判断。

b.应在“采取的措施”对应的项目上打。

c.监察员签名处应使用中文及拼音。如只有1名监察员参与检查，监察员2处的表格留空。

7.4.2处理流程

a.《发现问题通知书》应由进行检查的监察员在FSOP系统中使用中英文填写初稿，通知书格式见本手册附件D，并通过FSOP提交所在地区管理局外航处。

b.地区管理局外航处审核后，通过FSOP系统将《发现问题通知书》推送到外航对应的FSOP帐号和外航运行规范管理局外航处对应的FSOP帐号。同时系统邮件形式通知外航联系人登录FSOP系统查看发现问题的详细情况。

c. 外航运营人完成问题整改后，以FSOP系统推送方式反馈到负责数据收集和分析评估的监察员，如对整改项目无异义，则监察员关闭问题；否则通过FSOP系统推送意见，要求外航运营人作进一步整改。

7.4.3通知外航所在国民航局

a.运行规范管理局外航处应当收集监察员发现的重大问题，根据需要，向外航所在国民航局致函，并在FSOP系统备案。通报重大检查问题，致函前，应当征得民航局飞行标准司的认可。

b.致函格式参考本手册附件E。

7.4.4 行政约见和行政处罚

各地区管理局对外航进行行政约见和行政处罚措施应按照民航局有关行政执法要求实施。

# 8.外航检查参考用语

## 8.1检查用语样例

检查一般使用英语，有关英语样例参考见附件F。

## 8.2外航监察沟通障碍的应对

8.2.1外航机组成员无法正确理解监察员的提问

a.复述问题；

b.换一种提问方式；

c.直接将检查单适用部分提供给机组查看，简要写下所述内容。（例子：Minimum Equipment List / Medical Certificate/air operator certificate(AOC)）。

8.2.2监察员无法持续用英语表述

a.换一种提问方式；

b.寻求其他监察员帮助；

c.查手机英语词典；

d.动作、手势；

e.如果公司人员有能说中文，中文沟通。

8.2.3．监察员无法理解外航机组表述

a.要求对方复述；

b.寻求其他监察员帮助；

c.要求对方写下所述内容；

d.如果公司人员有人能说中文，中文沟通。

# 附件A  外航停机坪检查单

Flight operation

飞行运行部分

| 一、基本信息（用英文填写） |  |  |
| --- | --- | --- |
| Operator （运营人）: | Route from （进港航线）: | Scheduled/Nonscheduled Flight （定期/非定期）: |
| State of the Operator （运营人所在国）: | Inbound Flight No. （进港航班号）: | Aircraft Configuration(Passenger/Cargo) （客机/货机）: |
| Date of Inspection （检查日期/年月日）: | Route to （离港航线）: | Registration Marks （飞机注册号）: |
| Local Time （检查时间）: | Outbound Flight No. （离港航班号）: | Captain （机长）: |
| Airport （检查地点）: | Aircraft Type （飞机型号）: | First Officer （副驾驶）: |

|  | 二、检查项目 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Code 代码 | Item 项目 |  | Check 检查 | Focus on 检查重点 |  |
|  | A | Flight deck 驾驶舱 |  | S/合格 U/不合格 N/未观察 |  |  |
|  | 01※ | General condition 总体状况 |  |  | a.驾驶舱的清洁度、整齐度和总的情况 b.内部设备、行李箱、航图等的放置情况 c.驾驶舱舱门的锁定/开锁装置 d.驾驶舱窗户的状况，是否安装雨刷及是否可用 e.所有设备的安装没有明显不符合关于系统设计特点和紧急着陆的规定 f.电缆或电线是否裸露在外而未被发现 g.警告面板灯光是否工作 |  |
|  | 02 | Emergency exit 应急出口 |  |  | a.检查应急出口和绳索（如安装）的可用性 b.应急出口通道是否受阻或受限 |  |
|  | 03 | Equipment and requirement 运行设备和要求 |  |  | a. 对于运行所需设备，要求机组成员进行测试（如可行），包括但不限于：增强型近地警告系统TAWS（E-GPWS）、机载防撞系统ACAS II（TCAS）、驾驶舱语音记录器(CVR)、飞行数据记录器(FDR)、气象雷达、紧急定位发射机（ELT）， b.指定空域的运行要求（RVSM, PBN, 121.5频率的守听要求） |  |
|  | 04※ | Manuals 手册 |  |  | a.是否携带运行手册，飞机飞行手册（AFM）和其他必备的手册并且保持更新 b.机组人员是否能够理解手册所用语言 |  |
|  | 05 | Checklist 检查单 |  |  | a.检查单是否可用、易于获取并保持更新 b.检查单的内容是否与操作手册一致并涵盖所有的飞行阶段，包括正常和紧急情况下的操作 |  |
|  | 06※ | Radio Navigation Charts 无线电导航图 |  |  | a.飞行需要的航图（包含备降机场）是否便于取用，是否为最新修订版 b.导航数据库的有效性（如适用） |  |
|  | 07 | Minimum Equipment List (MEL) 最低设备清单 |  |  | a.核实是否有获得批准的最低设备清单并保持更新 |  |
|  | 08※ | AOC and Opspes or equivalent 运行合格证和运行规范或等效文件 |  |  | a.是否携带以及有效性、准确性 b.是否有英文版本 c.运行是否符合运行规范的限制,特殊授权（如LVO、PBN、RVSM、ETOPS等） |  |
|  | 09※ | Certificate of Registration 国籍登记证 |  |  | a.是否携带以及有效性、准确性 b.是否有英文版本 |  |
|  | 10※ | Radio license 无线电执照 |  |  | a. 是否携带以及有效性、准确性 b. 是否有英文版本 |  |
|  | 11※ | Certificate of Airworthiness 适航证 |  |  | a. 是否携带以及有效性、准确性 b. 是否有英文版本 |  |
|  | 12※ | Flight preparation 飞行准备 |  |  | a.是否有运行飞行计划（OFP）以及准确性 b.检查机组人员是否查阅可用的航行通告和气象信息,是否确认目的地机场或目的地备降机场的天气预报在最低运行标准之上 c.如果地面有结冰状况，检查是否已经采取适当的除冰措施或者是计划在飞机起飞前实施 d.飞行机组人员是否进行飞行前检查（包括外部绕机检查），飞机外部检查问题是否已得到机组确认 e.机组人员值勤期要求（符合运行手册要求） |  |
|  | 13 | Weight and balance sheet 载重平衡单 |  |  | a.是否有完整的载重配平单（纸质或者电子版），以及载重配平计算的准确性 |  |
|  | 14 | Hand fire extinguishers 手提灭火器 |  |  | a.是否放在指定位置且易于取用 b.数量、类型是否符合要求，是否标明具体使用方法 c.检查压力表（若已安装），检查有效期（若已标出） |  |
|  | 15 | Life jackets /flotation device 救生衣/漂浮设备 |  |  | a.是否有救生衣和漂浮设备（如适用），并易于取用 b.数量是否充足，是否可用 |  |
|  | 16 | Harness 安全带 |  |  | a.所有飞行机组成员是否都配有安全带并可用 b.安全带的状况（包括自动收紧装置） |  |
|  | 17 | Oxygen equipment 供氧设备 |  |  | a.是否配备供氧设备、是否可用 b.设备的状态，氧气压力，氧气面罩是否可以快速佩戴 |  |
|  | 18 | Emergency Flashlight 应急手电筒 |  |  | a.驾驶舱站位是否配备足够数量供机组使用的手电筒 b.夜间运行时,手电筒的状态和可用性 |  |
|  | 19※ | Flight crew license 飞行机组人员执照 |  |  | a.是否携带执照以及有效性，型别等级、仪表等级、熟练检查 b.是否携带体检合格证及有效性，检查备用视力矫正眼镜（如适用） c.检查执照中的英语水平等级资格 d.机组人员的配置，是否符合年龄要求 |  |
|  | 20※ | Journey log book,or equivalent 飞行记录本或等效文件 |  |  | a.是否有飞行记录本或等效文件 b.是否规范填写 |  |
|  | B | Cabin Safety 客舱安全 |  | S/合格 U/不合格 N/未观察 |  |  |
|  | 1 | General condition 总体状况 |  |  | a.客舱总体状况，包括洗手间、烟雾探测系统、易燃装饰物的状况 b.行李/设备储物箱以及洗手间是否放置了尖锐、过重及过硬物品 c.服务手推车的刹车效应 |  |
|  | 2 | Cabin attendant station/crew rest area 乘务员座椅/机组休息区 |  |  | a.检查客舱机组座椅/机组休息区及安全带（包括肩带）的总体状况以及可用性 b.检查救生衣（如适用）数量，以及是否方便取用 c.检查内话通信系统的可用性 |  |
|  | 3 | First aid/emergency medical kit 急救箱/应急医疗箱 |  |  | a.检查数量是否足够 b.检查医疗用品的状况、标识，以及是否易于取用 |  |
|  | 4※ | Hand fire extinguisher 手提灭火器 |  |  | a.检查手提灭火器数量是否足够，是否放在指定位置且易于取用，灭火剂型号是否适用舱室失火类型 b.检查是否标明手提灭火器的具体使用方法 c.检查手提灭火器压力表（若安装）和有效期（若标出） d.检查防护式呼吸装置（PBE）的数量、可用性，及是否易于取用 |  |
|  | 5※ | Life jackets/flotation devices 救生衣/漂浮设备 |  |  | a.是否有救生衣和漂浮设备，并易于取用（如适用） b.数量是否充足，是否可用 |  |
|  | 6 | Seat capacity,seat belt and seat condition 座椅数量，座椅安全带和座椅状况 |  |  | a.旅客数量是否超过可用的座椅数量 b.座椅和安全带、加长安全带的状况 |  |
|  | 7※ | Emergency exit, lighting and marking, emergency flashlights 紧急出口，照明设备和应急手电筒 |  |  | a.应急出口和甲板通道的指示、照明和标志情况 b.客舱站位为每名乘务员配备了应急手电筒及其状况 c.应急出口的通道是否被行李/座椅/小桌板阻碍 |  |
|  | 8※ | Slides/life rafts(as required),ELT 滑梯/救生筏（如适用）应急定位发射器 |  |  | a.检查滑梯/救生筏（如适用）的数量及可用性 b.检查应急定位发射机（ELT）的型号及可用性 |  |
|  | 9 | Oxygen supply 便携式供氧设备 |  |  | a.便携式氧气设备（如适用）是否在指定位置，操作说明是否正确标出 b.便携式氧气设备（如适用）的可用性和最低数量（根据MEL） |  |
|  | 10 | Safety instruction 安全须知 |  |  | a.如有安全须知卡，检查准确性，并确保数量充足、可用 b.如没有安全须知卡，核实是否有其他方式来传达要求的信息 c.检查“扣紧安全带”、“禁止吸烟”和“返回座椅（洗手间）”标识 |  |
|  | 11 | Cabin crew members 客舱乘务员 |  |  | a.数量是否满足的最低要求（查阅公司运行手册） b.是否熟悉客舱应急程序、应急设备的位置和操作 c.当给载有乘客的飞机加油时，检查在指定的位置上是否配备有合格的人员（依照运行手册程序），检查是否有能力与地面人员建立双向通信 d.检查客舱机组成员是否满足值勤期要求（符合运行手册要求） |  |
|  | 12 | Safety of baggage 行李安全 |  |  | a.检查机组和旅客是否未携带超出航空器储放能力的超大手提行李 b.检查机舱行李是否储放妥当 |  |
|  | 三、发现问题 Findings（使用中英文填写） |  |  |  |  |  |
|  | NO. 序号 | Code-focus on 代码-检查重点 | Detailed Description 问题详细描述 |  |  |  |
|  | 1 |  |  |  |  |  |
|  | 2 |  |  |  |  |  |
|  | 3 |  |  |  |  |  |
|  | 4 |  |  |  |  |  |
|  | 四、Remarks 备注（使用中文或英文填写） |  |  |  |  |  |

| 五、签名 |  |
| --- | --- |
| Inspected by（监察员1）: | Captain signature（机长签名）: |
| Inspected by（监察员2,如适用）: | Date（年月日）: |

检查单版本号：FO-R1

Airworthiness

适航维修部分

| 一、基本信息 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operator （运营人）: |  |  |  | Route from （进港航线）: |  |  |  |  | Scheduled/Nonscheduled Flight （定期/非定期）: |  |  |
| State of the Operator （运营人所在国）: |  |  |  | Inbound Flight No. （进港航班号）: |  |  |  |  | Aircraft Configuration(Passenger/Cargo) （客机/货机）: |  |  |
| Date of Inspection （检查日期/年月日）: |  |  |  | Route to （离港航线）: |  |  |  |  | Registration Marks （飞机注册号）: |  |  |
| Local Time （检查时间）: |  |  |  | Outbound Flight No. （离港航班号）: |  |  |  |  | Captain （机长）: |  |  |
| Place of Inspection （检查地点）: |  |  |  | Aircraft Type （飞机型号）: |  |  |  |  | First Officer （副驾驶）: |  |  |
|  |  | 二、检查项目 |  |  |  |  |  |  |  |  |  |
|  |  | Code 代码 | Item 项目 |  |  | Checked 检查 | Focus on 检查重点 |  |  |  |  |
|  |  | A | Flight deck 驾驶舱 |  |  | S/合格 U/不合格 N/未观察 |  |  |  |  |  |
|  |  | 07 | Minimum Equipment List 最低设备清单 |  |  |  | a.核实是否有最低设备清单 b.是否为最新版及获得批准 |  |  |  |  |
|  |  | 12※ | Certificate of Airworthiness 适航证 |  |  |  | a.是否携带以及有效性、准确性 b.是否有英文版本 |  |  |  |  |
|  |  | 20 | Journey log book,or equivalent 飞行记录本或等效文件 |  |  |  | a.是否有飞行记录本或等效文件 b.是否规范填写 |  |  |  |  |
|  |  | C | Aircraft condition 航空器状况 |  |  | S/合格 U/不合格 N/未观察 |  |  |  |  |  |
|  |  | 1※ | General external condition 外部总体状况 |  |  |  | a.机身的外部腐蚀，外观洁净度，是否存在冰、雪、霜冻现象，标识的清晰度 |  |  |  |  |
|  |  | 2 | Doors and hatches 舱门和舱口 |  |  |  | a.电缆状况，舱门外部标记及使用规范 b.舱门、舱口及相关密封圈的状况 |  |  |  |  |
|  |  | 3※ | Flight controls 飞行操纵舵面 |  |  |  | a.外部飞行操纵舵面 b.静电放电刷状况 c.电缆状况 |  |  |  |  |
|  |  | 4※ | Wheels,tyres and breaks 机轮、轮胎和刹车 |  |  |  | a.机轮和轮胎的损坏及磨损程度 b.轮胎气压是否正常（可行时） c.刹车系统状况 |  |  |  |  |
|  |  | 5 | Undercarriage 起落架 |  |  |  | a.标记、标牌的清晰度 b.检查润滑、泄漏和腐蚀情况，及起落架舱门连接件和铰链的磨损 d.起落架减震支柱镜面状况 |  |  |  |  |
|  |  | 6 | Wheel well 轮舱 |  |  |  | a.润滑、泄漏和腐蚀以及舱门接缝和铰链上的腐蚀状况 b.电缆状况 c.清洁及受损状况 |  |  |  |  |
|  |  | 7※ | Powerplant and pylon 发动机和吊架 |  |  |  | a.凹痕和松动或丢失的锁扣 b.低压涡轮、低压压气机桨叶（可视），传感器的明显损坏 c.裂纹 d.面板校准且把手齐平 e.异常损坏与泄漏 f.反推装置状况 g.进气道消音板的状况 h.标记与标牌的现状和清晰度 |  |  |  |  |
|  |  | 8 | Fan blades 风扇叶片 |  |  |  | a.风扇叶片损坏是否超出限度 |  |  |  |  |
|  |  | 9 | Propellers 螺旋桨,旋翼（主体/尾部） |  |  |  | a.腐蚀、轮毂处叶片松动、外来物造成的损坏等 b.除冰罩（接缝处）损坏 |  |  |  |  |
|  |  | 10 | Obvious previous repairs 明显的维修 |  |  |  | a.注意任何之前的维修，检查情况并核实是否符合标准做法 |  |  |  |  |
|  |  | 11※ | Obvious unrepaired damage 明显未经维修的损伤 |  |  |  | a.未评估和未记录的破损,包括腐蚀、雷击损坏、鸟击等 b.是否对所有破损都进行了观测、评估以及在损坏图/损坏扣和凹痕图中有所记录 |  |  |  |  |
|  |  | 12※ | Leakage 泄漏 |  |  |  | a.燃油、液压以及洗手间液体（蓝冰，如有）的泄漏情况 |  |  |  |  |
|  |  | 13 | Maintenance release and Defect notification and recertification 维修放行，故障通告和修理 |  |  |  | a.维修放行单是否已由授权人员签发 b.是否存在保留故障，是否符合飞机最低设备清单的要求 c.是否所有故障（较小故障，较大故障，凹坑，损坏等）已据实记录并经过评估 d.相应维修作业是否已据实记录，包括描述作业过程 |  |  |  |  |
|  |  | D | Cargo 货物 |  |  | S/合格 U/不合格 N/未观察 |  |  |  |  |  |
|  |  | 1※ | General condition of cargo compartment 货舱总体状况 |  |  |  | a.货舱的总体状况 b.照明、防火、火警探测系统和灭火系统状况（如适用） c.侧壁面板和舱顶板（卸压口盖），烟雾探测器，隔烟板/幕的状况 d.货物屏障/分割网及其状况 |  |  |  |  |
|  |  | 2 | Storage of cargo on board 机载货物的安全 |  |  |  | a.货物分装是否合理（考虑地面限制、高度限制、货盘和集装箱的最大总重量限制） b.货物是否安全装载 c.集装箱、货盘、锁具组件和捆货网绳的状况 d.货物划分区的网绳状况 |  |  |  |  |
|  |  | 三、Findings 发现问题（使用中英文填写） |  |  |  |  |  |  |  |  |  |
|  |  | Numbers 序号 | Code-focus on 代码-检查重点 |  | Detailed Description 问题详细描述 |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
|  |  | 四、Remarks 备注（使用中文/英文填写） |  |  |  |  |  |  |  |  |  |
| 五、签名 |  |  |  |  |  |  |  |  |  |  |  |
| Inspected by（监察员1）: |  |  |  |  |  |  |  | Captain signature（机长签名）: |  |  |  |
| Inspected by（监察员2，如适用）: |  |  |  |  |  |  |  | Date（年月日）: |  |  |  |

检查单版本号：AW-R1

# 附件B  检查项目指南

A 驾驶舱

A01总体状况 检查说明： a.检查总体状况。 b.检查内部设备、行李箱、航图等的配载情况。 注：监察员要有充分理由认定手册、机组行李箱等在即将进行的飞行中确实存放不当。例如可以证明（或合理推定）飞行中未放置手册缘于没有合适的存放区域。但是如果不能排除机组人员确实携带了手册，则不能给出检查结果。这些手册和箱子在监察员进入驾驶舱前的滑行阶段以及调头时可能确实由机组人员使用。 c.如果安装了驾驶舱舱门，应检查舱门的锁定/开锁装置。 最大起飞重量大于45500千克的客机（或载客数超过60人的客机）应特别检查驾驶舱加固舱门的安装情况和可用性。 d.检查是否可以从任一飞行员座位监控舱门外侧区域。有些监控方法，例如闭路电视，可以完全满足要求。然而，采用观察孔之类的方式可能导致机组人员无法从座位监控到舱门外侧区域，这种情况应给出2类检查结果。驾驶舱对舱门区域的可视监控非常重要。因此虽安装了观察孔但只监听声音也会认定为不符合要求因为声音监控没有图像。此时可给出2类检查结果。如果在飞行关键阶段这些问题得到纠正，比如专门由一名机组成员代表飞行机组对舱门外侧区域进行监控，或者禁止他人接近舱门外侧区域，这种情况下会给出一个检查结果，但由于对飞行安全影响相对较小（因此可使用1类结果）。在飞行的所有阶段都加配一名机组成员在驾驶舱可以认定完全符合国际民航组织的要求。 e.检查驾驶舱窗户的状况（例如挡风板裂痕，可能存在的脱层等）。 f.检查确定所有设备的安装没有明显不符合附件8第III部分A/B第4章中关于系统设计特点和紧急着陆的规定（例如安装在遮光罩上的设备严重影响飞行员的视野）。 g.检查挡风板雨刷是否安装及是否可用（如果此次飞行需要）。 h.检查电缆或电线是否裸露在外而未被发现。 i.检查警告面板灯光是否工作。 参考文件：附件6-I-1，附件6-I-2， 附件6-I-9，附件8-IIIA，附件8-IIIB

A02紧急出口 检查说明： a.检查紧急出口的可用性。如果安装绳索则检查绳索的安全性。 b.检查紧急出口通道是否受限或受阻。 注：监察员应该注意到在准备飞行时设备/行李可能暂时以不安全的状态放在安全出口。在这种情况下，监察员应该在起飞前对设备/行李是否已安全放置进行确认。如果机组人员未对此进行确认，监察员可以据此给出相应的结果。 参考文件：附件8-IIIA，附件8-IIIB，附件8-V

A03运行设备和要求 检查说明： a.增强型近地警告系统TAWS（E-GPWS）。 检查是否已安装，是否可用。如果不可用，检查是否已做出适当的故障保留处理（在ATLB{航空器技术日志，有时也称作TLB或简称为航空器Tech Log}中予以报告），是否仍在MEL放行限度内。核实所安装的近地警告系统具有前视地形防撞功能。如果发现地形数据库过期，再次对照最低设备清单确认放行条件。 如果运行测试可以由飞行员实施，则要求飞行员测试。 注：对于某些机型，这个测试不能由飞行员完成，仅能由维修人员完成：这并不会构成检查结果。 注：一些具有驾驶舱内化系统的飞机安装有近地警告系统，如SSOS或SPPZ（APBZ）的近地警告系统，这些系统并不符合国际民航组织对增强型近地警告系统的要求。只有具有前视地形防撞功能的SPRBZ达到了国际民航组织的要求。 如果发现飞机没有安装地形提示和警告系统（增强型近地警告系统），主管当局应考虑强制该飞机立即停止运营。这类飞机只允许实施非盈利飞行。 b.机载防撞系统ACAS II（TCAS）。 检查是否已安装并可使用。如果不可使用，检查是否已做出适当的故障保留处理（在ATLB{航空器技术日志，有时也称作TLB或简称为航空器Tech Log}中予以报告），是否仍在MEL放行限度内。 如果运行测试可以由飞行员实施，则要求飞行员测试。 注：对于某些机型，这个测试不能由飞行员完成，仅能由维修人员完成：这并不会构成检查结果。 如果发现飞机没有安装合适的 ACAS II/TCAS，主管当局应考虑强制该飞机立即停止运营。这类飞机只允许实施非盈利飞行。 对于适航证是2012年3月1日后签发的飞机，检查 ACAS II 的7.1版本软件是否已经安装。 c.驾驶舱语音记录仪(CVR) 如果运行测试可以由飞行员实施，则要求飞行员测试。 注：对于某些机型，这个测试不能由飞行员完成，仅能由维修人员完成：这并不会构成检查结果。 d.缩小最小垂直间隔RVSM 检查是否因设施不可用（如果不可用）导致飞机的最小垂直间隔功能丧失（参见第9614号文件）。 e.基于性能的导航PBN 检查飞机是否安装区域导航设备。对于在B-RNAV 或 P-RNAV指定空域运行的飞机检查是否满足所需导航性能的要求。 f.121.5频率的守听 检查飞机通讯设备是否满足在运行当中守听121.5频率的要求，同时可以要求机组证实是否执行了监听121.5频率要求（如公司运行手册的相关描述）。 g.经中国民航认可批准的特殊运行（如高高原运行等），根据CCAR-129的相关规定，检查外航的相关运行是否获得中国局方的批准。 参考文件：CCAR-129和运行规范，附件6-I-6，附件2-2

A04手册 检查说明： a.检查是否携带运行手册和飞机飞行手册（AFM）。（注：飞行手册的资料可能包含在运行手册中）。 注：并非所有的运行手册都要携带上飞机，但至少应携带与飞行运行相关的部分。 检查手册的内容是否符合要求并且保持更新（例如：最新版本的AFM）。 注：尤其需要检查运行手册中的如下条目： ——是否携带关于重量配平控制的操作指南和数据 ——关于所携带导航设备的清单，包括任何有关基于性能导航所需的设备 ——携带可供机组人员计算性能的数据 ——燃油规划信息 ——飞行时间和值勤时间的要求 ——乘客在机上时补给燃油的安全措施 b.确保机组人员能理解运行手册和/或飞机飞行手册所用语言。 注：国际民航组织的标准并没有要求手册必须用英语书写。除非飞行员完全不明白手册所用的语言，一般情况不会构成检查结果。 注：如果机组人员中只有一位或部分成员不理解运行手册的语言，并且不会对安全产生影响，则可以相应给出2类或3类检查结果。 注：附件6的确要求运行手册的特定部分必须由国家权力机构批准。然而，附件并未要求将批准的证明文件加入手册中。这取决于每个缔约国认定批准的方式以及是否要求将批准的证明文件加入手册中。缺少证明文件并不会构成检查结果。 参考文件：附件6-I，附件18-9

A05检查单 检查说明： a.检查检查单是否可用并容易获取。 注：大多数的现代飞机都配备有电子检查单，例如空客公司的ECAM系统。如果机组人员能够证明他们有权使用这些检查单，并且这些检查单都被准确无误的记录在运行手册中，那么这并不构成检查结果。 b.检查运行手册是否包括规定的检查单。将机组人员使用的检查单与手册中的相关内容进行对照。 c.检查检查单的内容是否与操作手册一致涵盖所有的飞行阶段，包括正常情况和紧急情况的操作。 注：正常、非正常和紧急情况的检查清单有时可合并为“快速检查单”。不过，监察员可能会发现各个飞行阶段的独立检查单，这是完全允许的。 d.检查飞行机组所有人员持有的检查单是否相同。 注：如果检查单显示是不同的修订版本，或者不同的日期，则检查检查单的内容是否相同。 注：对于一些前苏联生产的飞机，只有随机工程师才有检查单。驾驶员可能仅凭记忆中的检查单进行检查。 参考文件：附件6-I

A06无线电导航图 检查说明： a.检查飞行需要的离场、航路、进近和机场航图是否便于取用，是否及时更新为AIP航图最新修订版（包含备降机场的相关信息）。 注：如果这些航图涵盖了所飞航线，或者将要飞行的航线，包括与之相关的改航，已及时更新为AIP航图最新修订版，那么航图资料库中一两条修订缺失是可以接受的。 注：如果没有更新其他图表，但是所需图表已更新，这种情况并不构成检查结果。 b.检查FMS（飞行管理系统）/GPS（全球卫星定位系统 ）数据库的有效性；如过期，检查MEL（最低设备清单）。 参考文件：附件6-I，附件15-6

A07最低设备清单 检查说明： a.检查最低设备清单（MEL）是否可用。 注：越来越多的运营人不在飞机上放置最低设备清单（MEL），但是可以通过数据下载链接获得。这是可以接受的方案。 b.检查最低设备清单（MEL）是否低于主最低设备清单（MMEL）。 注：如果发现最低设备清单（MEL）不包含最新版本的主最低设备清单（MMEL），这种情况应该按照其他类检查结果报告。 注：检查最低设备清单（MEL）的修订状态并不充分；如果最新修订推出了较低的限制条件，最低设备清单（MEL）可能无需更新。缺失的修订编号不会构成检查结果。文档管理流程要由监管机构批准。如果发现最低设备清单（MEL）未更新而导致产生一份较低限制的文件，可以在有关文档管理流程的后续阶段提出问题。 注：将更为严格的要求引入主最低设备清单（MMEL）需要一定的时间，监察员要为主最低设备清单（MMEL）的修订设置至少4个月的时间框架（自修订的最低设备清单（MEL）公布起）。 c.检查最低设备清单（MEL）目录是否反映出在飞机上实际安装的设备，并考虑到在运行规范里的特殊审批。检查最低设备清单（MEL）是否包含了维修和/或运行程序。 d.检查最低设备清单（MEL）是否客户化。例如,MEL不应该包含一个控制部件的参考数量(“ATA 23通讯系统——任何超过CFR14要求的，不是由备用汇流条供电并在紧急程序中没有要求的，可以不工作),但应提到实际所需的数量 注:主要为客舱相关设备,数量可能会缺失,只要MEL反映了一个可以替代的控制方法。 e.检查保留故障是否与最低设备清单所指示的一致。 注：附件6的确要求运行手册的特定部分必须由国家权力机构批准。然而，附件并未要求将批准的证明文件加入手册中。这取决于每个缔约国认定批准的方式以及是否要求将批准的证明文件加入手册中。缺少证明文件并不会构成检查结果。 参考文件：附件6-I

A08 航空运营人合格证和运行规范或等效文件 检查说明： a.检查文件是否携带以及文件的精确性（包括运行规范） b.检查AOC的格式（结构和内容）是否与附件6的规定（包括以另一种语言写的文件要有英语译文）一致。 注：尽管国际民航组织对结构有明确要求，但如果内容符合国际民航组织的要求只是结构不同，并不构成检查结果。 c.检查飞机运行(入港和出港)是否符合运行规范(限制、特殊授权：LVO（低能见度操作）、PBN（基于性能的导航）、RVSM（缩小最小垂直间隔）、ETOPS（双发飞机延程运行）以及其他为执行特定类型运行的要求)。 注：如果飞机上没有发现AOC和/或运行规范，应使用3类预设问题处理。如果在检查时没有提供规定的文件，飞机仍能放行从事非商业性通用航空飞行。但是，如果在后续过程中获得适当证据表明检查时已有签发的有效文件，可降级为1类检查结果。 参考文件：附件6-I

A09国籍登记证 检查说明： a.检查是否携带以及准确性。如果机上只有照片复印件，应以“没有有效登记证或机组未能出示证书”为由，给出检查结果。 b.检查其格式和内容是否符合要求，是否翻译成英文。 c.检查防火标识板（通常在左前门附近），并将面板上的数据与登记证上的数据进行比较。 注：附件7中规定，在主要入口附近需要安装一个防火板。通常防火板也安装于飞机其他位置。虽然这不符合要求，但是对安全影响很小，因此没有必要给出检查结果。 注：虽然国际民航组织并没有明确规定携带原始文件的副本，但是机上携带发证机关认证的复印件也可以接受。 注：如果在检查过程中未在飞机上发现登记证，应使用2类预设问题反映这种情况。但是，如果在后续过程中得到适当的证据表明检查时已签发有效的文件，可以降级为1类检查结果。 参考文件：附件7，芝加哥公约-29a

A10无线电执照 检查重点： a.检查是否携带以及文件的准确性。 b.检查名称和呼号是否正确。 注：根据《芝加哥公约》第29e条和第30条，无线电执照是指允许安装无线电发射装置的许可证。国际民航组织没有具体说明无线电执照应包括的信息。拥有无线电执照的要求起源于国际电信联盟《无线电管理条例》第十八条的规定，要求发行执照的国家，其执照上应具有名称、呼号，还应具有“安装的一般特点”。 然而，这种执照的确切内容由国际电信联盟给出，而且仅仅作为建议(推荐7Rev.WRC-97)。 因此不能根据无线电执照的内容认定检查结果，除非所提到的信息不正确。 注：虽然国际民航组织并没有明确允许携带原始文件的副本，但是机上携带发证机关认证的复印件也可以接受。 注：当飞机实施商业运行时，在检查过程中监察员如果发现无线电执照没有被带上飞机，应按1类记录检查结果。 注：某些无线电执照有有效期。如果发现无线电执照已经过期，应按1类记录检查结果。 参考文件：芝加哥公约-29e，芝加哥公约-30a

A11适航证 检查说明： a.检查是否携带以及文件的准确性和合法性。 b.检查适航证的内容是否符合要求（包括用另一种语言编写的相应英语译文）。 注：如果飞机被核实没有有效的适航证原件（或经认证的复印件），则认定为3类检查结果。这架飞机只有在收到登记国确认飞机拥有有效适航证的情况下，才能被批准离场。 注：某些国家(如EASA成员国)发行适航证时，没有提及有效期。这类证书通常会由一个独立的文档加补充(ARC-适航审查证书)，来表明其有效性。 注：虽然国际民航组织并没有明确允许携带除了原来的文件的其他文件，但是如果一个由发证机关认证的复印件被携带上飞机上时，被认为是可以接受的。 注：如果在检查过程中没有在飞机上发现适航证，应记录为3类问题。但是，如果在后续的过程中得到适当的证据表明检查时已有有效的文件，可将检查结果降级1类。 参考文件：附件8-II，芝加哥公约-29b，芝加哥公约-31，芝加哥公约-39a，芝加哥公约-40

A12飞行准备 检查说明： a.检查是否有运行飞行计划，是否准确（包括机长的署名）。与运行手册相关说明进行比较。 b.检查文件系统（包括与飞行准备相关的所有文件）、燃油及性能计算。 注：如果机载的实际燃油比预计的要多，但考虑到性能、载重和配平计算的问题，这不应该认定为检查结果。检查结果应在性能和/或者载重和配平计算时提出。 c.检查即将飞行的飞机的燃油消耗监控（如果运行手册对此有要求）。 d.检查飞行员是否已经选择了合适的备降机场（如有要求）。 e.检查飞机机组人员是否已经查阅了所有气象信息（包括备降机场）。 注：根据前一条注释，附件6-I-4.3.5.2未规定信息需要带上飞机。监察员应确认飞机离港飞往国外之前机组人员已获得这些信息。 f.检查机组是否确认目的地机场或目的地备降机场的天气预报在最低标准之上。 g.检查机组人员是否查阅可用的航行通告，以及/或者飞行前信息公告（包括备用机场信息）。 注：根据附件6-I-4.1.1，运营人/飞行人员必须知道地面以及/或者水上设施的可用性（通常发布在航行通告里）。如果机组人员知道，就不必要求把航行通告携带上机并且不会得出检查结果。为了确认机组人员（在航行通告没有在飞机上的情况下）确实清楚信息，监察员可以针对目的地机场或目的地备降机场发布的航行通告信息进行检查）。 注：根据修订要求，拥有飞行签派部门的运营人，可以仅为机组提供飞行所需的航行通告。 如果地面有结冰状况，检查是否已经采取适当的除冰措施或者是计划在飞机起飞前实施。 h.检查空中交通管制飞行计划是否存在及其准确性。 注：备用机场并不总是需要在ATC飞行计划中列出。例如，允许没有备用机场或往返飞行（RPL）的飞行。如果是往返飞行，飞行计划中应当提及联系方式，以确定ATC能够获取相关飞行所选定的备用机场信息。（见第4444号文件） 注：根据运行类型，飞行计划第10项中应当包含以下指示符： “R”代表 B-RNAV运行； “P”代表P-RNAV运行； “Y”表示在空域或部分航路上要求强制使用8.33千赫频率间隔可用无线电设备的飞行； “W”代表RVSM运行； “S”表示携带S模式应答机的飞机。 i.核查是否已进行飞行前检查及其他等效检查，检查结果是否正式确认。 参考文件：附件6-I，附件2-3

A13载重平衡单 检查说明： a.检查是否有完整的载重配平单（纸质或者电子版），以及载重配平计算的精确度。 b.检查实际的业载分配是否如实反映在M&B表格中。 如果发现载重和/或配平的计算不准确，检查是否在飞机限度内，并且检查对性能计算的影响。 注：如果装载额外燃油，确认将其加入载重配平文件中。 c.检查机组人员是否有可用的有效数据（在OPS手册中或者AFM中）来确认载重配平计算。 d.检查载重配平计算是否说明因噪声规定而降低最大起飞重量的飞行限制。 参考文件：附件6-I

A14手提灭火器 检查说明： a.检查灭火器是否放在指定位置，是否便于取放。 b.检查灭火器是否标明相关使用说明。 c.检查灭火器是否可用，检查失效日期（如果有）。如果明显重量很轻则考虑认定灭火器不可用。 参考文件：附件6-I，附件8-IIIA，附件8-IIIB，附件8-V

A15救生衣/漂浮设备 检查说明： a.检查是否有救生衣和漂浮设备，易于取用，数量是否充足，是否可用。 注：ICAO并未规定救生衣要有有效期（或下一次检查日期）。运营人要以采用不同的系统监控救生衣的状况。没有日期的救生衣或漂浮设备并不构成检查结果。但是，如果显示已过期（或下一次检查日期），则考虑认定不可用。 注：ICAO仅要求跨水运行携带救生衣和漂浮设备（详见附件6相关内容）。如果国内或国际飞行都不是跨水运行，则不应据此给出检查结果。 参考文件：附件6-I

A16安全带 检查说明： a.检查飞行机组全体成员是否都配有安全带并可用。 b.检查安全带的工作性能（包括自动收紧装置）。如果不可用，与MEL对照检查。 注：如果安全带的功能被座椅套限制，考虑认定安全带此项功能不工作。 注：如果自动收紧装备不工作，考虑认定安全带不工作。 注：座椅安全带并不符合ICAO的安全带要求，应考虑认定未安装安全带。 参考文件：附件6-I

A17供氧设备 检查说明： a.检查是否配备供氧设备、是否可用以及设备的状态。 b.检查氧气面罩是否可以快速佩戴。 注：ICAO没有提供“快速佩戴”面罩的具体定义，因此，如果发现这方面存在问题，监察员必须谨慎地行使权利。那些不符合FAA标准（一只手可以不超过5秒把面罩佩戴到脸上···）的面罩只能认定为普通面罩。然而，如果机组人员不能够证明以下内容，可以认定为缺少可快速佩戴的面罩： ——面罩足够所有机组成员使用； ——面罩可以进行无线电通讯； ——面罩对戴眼镜的机组成员不造成障碍。 c.检查氧气瓶压力，如果是低压，根据OPS说明手册检查最低限压，可以要求机组人员对带有无线电通讯功能的氧气面置进行运行状态检查，从而确认其状态。 注：ICAO没有规定氧气面罩或氧气瓶的有效期。运营人可以采用各种系统来监控氧气面罩的状态。氧气面罩或氧气瓶没有生产日期并不能得出违规的结论。但是，如果已过失效日期（或下一个检查日期），应认定为不可用。 参考文件：附件8-IIIA，附件8-IIIB，附件6-I

A18应急手电筒 检查说明： a.检查是否所有机组成员都配备有合适的手电筒。 b.检查手电筒的状态、可用性和开关。注意日间起飞但一直飞到晚上的航班也必须符合要求。 注:只有夜间航班需要为机组人员配备手电筒。包括日间起飞持续到晚上的航班以及晚上起飞白天到达的航班。在检查全程都是日间飞行的航班时，手电筒缺少并不能得出检查结果，但是应当计入其他问题。 注：如果手电筒的正常功能受到电池电量低的严重影响，则认定为不可用。 注：如果只有私人手电筒可用，同时能够在正常的位置供机组人员使用，这并不能得出检查结果，然而这也应当计入其他问题。 参考文件：附件8-IIIA，附件8-IIIB ，附件6-I

A19飞行机组人员执照 检查说明： a.检查机组人员的执照是否携带，是否有效，是否处于合适的等级。 b.检查体检合格证是否携带及其有效性。 c.检查格式与内容(包括英文翻译)是否符合国际民航组织附件1的要求。如果执照是由相关机构而非登记国颁发，检查执照的有效性（是否经过确认）。 d.检查备用的视力矫正眼镜（查看机组人员是否佩戴了合适的执照） e.检查执照中的英语水平等级。 注：明确表述执照中的ELP水平的规定并非是必须的，这种情况不得给出检查结果。然而，如果表明等级低于4级则应当给出检查结果。这同样适用于4级、5级签注失效，同样要给出检查结果。 注：ICAO鼓励缔约国采取灵活的方法，采用语言熟练性实施计划取得进展。因此，对于这些国家不能给出3类检查结果。对于没有签订正式行动文件的国家，如果发现执照上英语等级不合格应该给出3类检查结果。 注：如果在检查中发现飞行员英语等级合格但是明显存在英语交流困难，应认定为其他类问题。 注：体检评估的1、2、3级可以通过不同方式认证，比如，适当的独立证明，执照的状态，国家机构规定的健康评估是执照上不可缺少的一部分。 注：飞行机组人员执照复印件是不符合ICAO要求的，除非机组可以证明其持有的执照正处于更新或换照的状态，这种情况下是不应该给出检查结果的。 注：如果飞行机组人员的执照在检查期间没有携带，可以记录为3类问题。然而通过合适的证据可以证明在飞机起飞前机组人员的确持有合适有效的执照，只是没有携带而已，可以记录为1类结果“检查期间飞行机组人员持有合适的执照但是没有随身携带”。如果在飞机起飞前证据不充足，则应该记录为3类检查结果“飞行机组人员没有合适的执照”，并且要求采取纠正措施。任何情况下，在没有获得确认飞行机组持有有效的执照之前，任何机组成员都不得获得执行飞行任务的批准。 注：在检查欧洲飞行机组人员执照的时候，监察员必须考虑到不同欧洲国家之间执照的相互认证，持有由任意一家EASA发放执照的人员在不需要任何额外验证的情况下就能在任意一架由EASA注册的飞机上享受相应权力。 如果外界环境的影响明显占主观因素（例如飞机严重延误），检查机组人员是否依然遵守运行手册中有关飞行时间和值勤时间的规定。 f.检查机组人员是否符合年龄要求（飞行员是否超过60岁） 参考文件：附件1-1，附件6-I，芝加哥公约-29，芝加哥公约-32，芝加哥公约-39b，芝加哥公约-40

A20 飞行记录本或等效文件 检查说明： a.检查是否有飞行记录本。 注：某些情况下，飞行记录本可以由“一般报告”（general declaration）替代（包括附件6第Ⅰ部分11.4.1款列出的信息）。 b.检查飞行记录本/一般报告是否符合要求，是否正确填写。 参考文件：附件6-I， 芝加哥公约-34 ，芝加哥公约-29d

B客舱

B01内部整体状况 检查说明： a.检查整体状况，包括洗手间、烟雾探测系统、舱顶板储藏槽以及易燃装饰物的状况。 b.检查行李/设备贮物箱，以及洗手间有无放置尖锐、过重及过硬物品（在已上锁洗手间内临时存放垃圾袋是可接受的）。 c.检查在2005年12月4日以后生产的服务手推车的刹车效应。 注意：只有当刹车效应明显不符合标准的时候才会在检查结果中提及。存在刹车故障的手推车可以在适当标记后放在厨房内作为储物手推车使用。 参考文件：附件8-IIIA，附件8-IIIB，附件6-I

B02客舱乘务员座椅/机组休息区 检查说明： a.检查客舱机组人员座椅的总体状况以及可用性。 注：如果客舱机组人员座椅被检查出与MEL所规定的可用性不符，就需检查可用座椅的数量是否满足客舱机组人员所需座椅数量的最低需求（详情参阅运行手册）。 注：如果发现客舱机组人员座椅不能自动收回从而对飞机在发生紧急情况时的快速疏散工作构成阻碍，就需将此调查结果记录在项目B07“紧急出口”中。 b.检查肩带和安全带的现状和使用状况。 注：在1981年1月1号当日或之后发布单机适航证的飞机必须为客舱乘务人员座位配上安全肩带。 c.检查救生衣能否方便取用。 d.检查通信系统（驾驶舱-客舱以及客舱-客舱）的可用性。若不可用，则根据MEL进行检查。 参考文件：附件6-I

B03急救/应急医疗箱 检查说明： a.检查医疗用品的现状、标识及是否易于取用。 注：ICAO只对急救箱、应急医疗箱和卫生防疫包的配备提供建议。 注：ICAO未规定急救箱、应急医疗箱、卫生防疫包必须标明有效期（或下次检查日期）。急救箱、应急医疗箱和卫生防疫包的有效期缺失不构成检查结果。但是，如果发现在已标明的有效期外使用这三种设备，就需将其作为检查结果上报。 参考文件：附件6-I

B04手提灭火器 检查说明： a.检查灭火器是否放在指定位置，是否便于取放。 b.检查灭火器是否标明相关使用说明。 c.检查灭火器是否可用，检查失效日期（如果有）。如果明显重量很轻则考虑认定灭火器不可用。 参考文件：附件6-I，附件8-IIIA，附件8-IIIB，附件8-V

B05救生衣/漂浮设备 检查说明： a.检查是否有救生衣和漂浮设备，易于取用，数量是否充足，是否可用。 注：ICAO并未规定救生衣要有有效期（或下一次检查日期）。运营人要以采用不同的系统监控救生衣的状况。没有日期的救生衣或漂浮设备并不构成检查结果。但是，如果显示已过期（或下一次检查日期），则考虑认定不可用。 注：ICAO仅要求跨水运行携带救生衣和漂浮设备（详见附件6的相关内容）。如果国内或国际飞行都不是跨水运行，则不应据此给出检查结果。 参考文件：附件6-I

B06座椅数量，座椅安全带和座椅状况 检查说明： a.检查可使用的座椅数量。 b.检查座椅和安全带现状 c.检查加长安全带的可用性和状况（如需要） 参考文件：附件6-I，附件8-IIIB

B07紧急出口，照明设备和应急手电筒 检查说明： a.检查紧急出口标志，照明设备及手电筒状况。 b.检查撤离通道照明设备状况。 c.检查在充满烟雾的客舱内的紧急出口目视指示的现状。 d.检查紧急出口操作标识的现状。 注：应提醒监察员，在充满烟雾的客舱内，被照明的撤离通道和紧急出口目视指示是有区别的。1960年7月13日之前申请合格审定的最大起飞重量超过5700KG的飞机不要求装备撤离通道照明设备和出口照明设备。2004年3月2日之前申请合格审定的超过5700千克的飞机无需在充满烟雾的客舱内标明紧急出口目视指示。如果已使用目视照明指示系统，通过安装低层照明或荧光发光系统，则两种要求均能满足。虽然ICAO对大多数飞机是否应标明目视指示不做要求，但大多数飞机已将这些指示标出。如果指示设备发生任何故障，应根据MEL进行维修，出具检查结果时也应参考MEL。 e.检查地板/地毯的状况。 f.检查应急出口的通道是否被行李/座椅/小桌板阻碍。 注：某些类型的紧急出口可能过大。在此种出口旁安置座椅不构成检查结果。只要剩余空间能达到规定尺寸的最低标准，就无需提交检查结果。 注：在紧急出口前的座椅排不能倾斜。不过如果在座椅后没有其他应急出口，那这些临近出口的座椅就可以倾斜。 注：如果小桌板扣在受减速力或冲击力影响时无法扣住小桌板使其保持直立状态，就应将其作为检查结果上报。不过检查结果的等级还是取决于相关小桌板的位置（是否靠近应急出口）。 注意：（取决于检定标准）某些机型应在紧急出口附近的座椅上装备特定的小桌板（单向或嵌入式锁扣），以防止在撤离飞机时无意间碰落小桌板。只有那些未装备特定小桌板的飞机才构成检查结果。当检定这些问题时，监察员需要格外谨慎。 g.检查所有机组成员站位配备的手电筒是否可用。 h.检查手电筒状态、可用性以及是否易于取用。白天离港夜间持续飞行的航班应满足此要求。 注意:只有在夜间运行的飞机要求机组配备电子手电筒。包括白天起飞夜间持续飞行的航班，和夜间起飞白天到达的航班。如果飞机仅在白天飞行，则任何手电筒的缺失或非适用性将不构成检查结果。不过也应将其作为其他类检查结果上报。 注：如果手电筒因为电量微弱而无法正常使用，则认为它不可用。 注：如果仅有个人的手电筒可用，则不构成检查结果，只要它们摆放位置正确且机组成员能方便取用。不过也应将其作为其他类检查结果上报。 参考文件：附件8-IIIA，附件8-IIIB，附件6-I，附件8-V-F

B08滑梯/救生筏（如适用），应急定位发射机 检查说明： a.检查滑梯/救生筏的数量和可用性。 注：滑梯/救生筏的可用性可以通过检查压力表（如已配备），或有效期（或下一次检查日期）来检验，如果已过有效期（或下一次检查日期），就判定为不可用并按MEL进行调查。 注：ICAO要求只有水上飞行的飞机才能配备漂浮设备（参考下文附件6）如果进港航班离港航班或者其他航班均不进行水上飞行，那么此检查项目不应被上报为调查结果。 b.检查ELT的当前型号和可用性。 对ELT进行校验，使其在406 MHz的频率进行广播，相关数据可在ELT上（如果是便携式）、飞机无线电台许可证上（尽管并未要求将频率标在此处）或者使用手册上（包括在紧急救生设备清单里）找到。 注：如果无法找到ELT广播频率的数据，应将其作为其他类问题上报。 注：如果ELT无法在406 MHz 和121.5MHZ上连续发射，应将其作为其他类问题上报。 注：ICAO标准中提到的“首次颁发单机适航证”应理解为飞机生产后交付的首个适航性证明。 c.检查火灾求救信号设备（如有要求并且易于操作的话）。 参考文件：附件6-I，附件8-IIIA，附件8-IIIB，A10-III-Ch，附件8-V-F

B09供氧设备 检查说明： a.检查便携式呼吸设备是否在指定位置，操作说明是否正确标出。 b.使用储氧后要检查客舱氧气量（通过压力表或电子显示器）。 c.检查便携式呼吸设备的可用性和最低数量（根据MEL）。 d.检查氧气分配装置或氧气面罩（当可能时）的数量和可用性。 注：如果氧气面罩和氧气瓶接口不兼容的话，认为氧气面罩不可用。标准大气层中与本文所用的绝对压强相对应的大概高度如下： 注：ICAO则要求飞机上携带的氧气量要足够所有乘客使用。 参考文件：附件6-I，附件8-IIIA，附件8-V-F，附件8-IIIB

B10安全须知 检查说明： 如果飞机上有飞行简令卡，请检查它们的准确性，并确保可用和数量充足。 如果飞机上没有飞行简令卡，请核实是否有其他方式来传达要求的信息。 注：ICAO要求将某些安全相关信息传达给乘客。传达方式可以由运营人决定（飞行简令卡、口头阐述、视频展示，或这些方法的组合）。因此，乘客简介卡不会一直出现在飞机上或包含在所有相关安全信息内，这并不会构成检查结果，除非有证据表明运营人未将相关信息传递给乘客。 a.检查“系好安全带”和“返回座椅（洗手间）”标识。如果不可用，请参照MEL中的相关规定进行检查。 参考文件：附件6-I

B11客舱机组成员 检查说明： a.检查客舱乘务员构成是否满足机组构成的最低需求（符合运行手册要求）。 b.检查客舱乘务员是否熟悉客舱应急程序、应急设备的位置和操作。 当给载有乘客的飞机加油时，检查在指定的位置上是否配备有合格的人员（符合运行手册要求的程序）。此外，还要检查与地面人员所建立的双向通信设备。当外界环境（如重大延误）成为影响飞行的主导因素时，检查乘务人员是否遵守运行手册规定的值勤期要求。 参考文件：附件6-I

B12行李安全 检查重点： a.检查机组和旅客是否携带超出航空器储放能力的超大（超重）手提行李，检查机舱行李是否储放妥当。 参考文件：附件6-I

C飞机适航状况

C01外部总体状况 检查说明： a.检查机身的总体外部状况： ——腐蚀； ——外观洁净度； ——是否存在冰、雪、霜冻现象； ——标识的清晰度。 注：尽管遗漏下翼面注册标识是不符合国际要求的，但此问题的安全相关性较低。因此这个问题应该记录为其他问题。 注：标记用语可能是其他语言而非英语。 注：国际民航组织不要求标记破开点（如果要使用标记，则应该符合一定格式）。 注：在检查标记和标牌时，监察员应区分国际民航组织要求的标记和仅由制造商要求的标记。 ——锁扣和铆钉松动或丢失； ——天线的状况； ——静电放电器状况； ——外部灯光的状态及其是否可用等； 注：在上报检查结果之前，监察员应确保对灯光系统的检查符合航班运行种类的要求（依据MEL）。不可用的灯光系统如果不是此次航班运行需要的必要设备，则只记录为其他问题。 参考文件：附件8-IIIA，附件8-IIIB，附件6-I

C02舱门和舱口 检查说明： ——电缆状况； ——舱门外部标记及使用规范； 注：只有能从外部打开的舱门才需要外部标记； ——舱门、舱口及相关密封圈的状况。 参考文件：附件8-IIIA

C03飞行操纵 检查说明： a.检查外部飞行操纵舵面。 b.检查液压泄漏。 c.检查静电放电器状况。 d.检查电缆状况。 参考文件：附件8-II

C04机轮，轮胎和刹车 检查说明： a.检查机轮和轮胎的损坏及磨损程度。 b.检查轮胎气压是否正常（可行时）。 c.检查刹车系统状况。 d.检查机轮减摆器的状况。 注：有些飞机制造商可能允许飞机轮胎或刹车磨损或损坏程度超出AMM限制后进行一定数量的飞行。 参考文件：无

C05起落架 检查说明： a.检查标记、标牌的清晰度。 注：在检查标记和标牌时，监察人员应该区分国际民航组织要求的标记和仅由制造商要求的标记。 b.检查状态、润滑、腐蚀、漏油、损坏以及起落架减震支柱的不正常的伸展。 参考文件：附件8-IIIA

C06轮舱 检查说明： a.检查润滑、泄漏&腐蚀状况。 b.检查润滑、泄漏&腐蚀以及舱门接缝和铰链上的腐蚀状况。 c.检查电缆状况。 d.检查清洁及受损状况。 参考文件：无

C07发动机和吊架 检查说明： a.凹痕和松动或丢失的锁扣； b.低压涡轮、低压压气机桨叶（可视），传感器的明显损坏； c.裂纹； d.面板状态良好且锁定； e.异常损坏与泄漏； f.推力反推装置状况； g.进气道消音板的状况； h.标记与标牌的现状和清晰度。 注：在检查标记和标牌时，监察员应该区分国际民航组织要求的标记和仅由制造商要求的标记。 参考文件：附件8-IIIA，附件8-IIIB，附件8-V

C08风扇叶片 检查说明： a.检查外来物损伤、裂纹、割纹、腐蚀、侵蚀等。 参考文件：无

C09螺旋桨，旋翼（主体/尾部） 检查说明： a.检查腐蚀、轮毂处叶片松动、石击造成的损坏等。 b.检查除冰罩（接缝处）损坏。 参考文件：无

C10明显的维修 检查说明： a.检查针对设计异常或性能不良的修理是否符合标准。 注：没有要求在飞机上保存临时修理的信息（例如：关于凹痕和锁扣的修理）。但是机长必须了解飞机的状态以确保飞机符合适航性的要求。 参考文件：附件6-I

C11明显未经维修的损伤 检查说明： a.检查未评估和未记录的破损,包括腐蚀、雷击损坏、鸟击等。 b.检查是否对所有破损都进行了观测、评估以及记录。 参考文件：无

C12泄漏 检查说明： a.检查燃油、液压油以及（如果可行）洗手间液体（蓝冰）的泄漏情况。 注：在这些检验项目中，当C03, C04, C05, C06 以及C07项确认有泄漏情况发生，必须将其作为检查结果上报。 参考文件：无

C13维修放行，故障通告和修理 检查说明： a.检查机长是否确认维修放行单已经签发（常通过验收飞机） 注：定期维修后的维修放行单并不要求携带上机。检查机长如何确保飞机适航且维修放行单已签发。 b.核查是否存在保留故障（如有必要在报告中具体说明） c.核查是否所有故障（较小故障，较大故障，凹坑，损坏等）已据实报告并经过评估。 d.检查相应维修作业是否已据实报告，包括描述作业过程，AMM/SRM参考等。如果保留故障包括时限件，检查保留故障是否仍然存在。 e.检查是否符合飞机最低设备清单（MEL），如可行。 检查ATLB中所规定的修正间隔未超过MEL的要求。 注：没有要求记入ATLB的内容要用指定语言。但必须保证以飞行机组成员能够理解的语言录入ATLB。 参考文件：附件6-1

D货物

D01货舱总体状况 检查说明： a.检查货舱的总体状况。 b.检查照明、防火、检测以及灭火系统状况（如果情况允许）。 c.检查侧壁面板和舱顶板（卸压口盖），烟雾探测器，隔烟板/幕的状况。 d.检查货物屏障/分割网及其状况。 参考文件：附件8-IIIA，附件8-IIIB，附件8-V，附件8-II

D02机载货物的安全 检查说明： a.检查货物分装是否合理（考虑地面限制、高度限制、货盘和集装箱的最大总重量限制）。 注：并非所有飞机都有装载高度限制。 b.检查航班工具箱或飞行工具箱及备用轮胎是否正确装备。 c.检查货物是否安全装载。 d.检查集装箱、货盘、锁具组件和捆货网绳的状况。 e.检查货物划分区的网绳状况。 注：虽然大部分情况下货物是由捆货网绳固定的，但有些飞机没有装备这样的捆货网绳，而是将货物放置在货物隔间位置来达到固定货物的目的。（例如：部分涡轮螺旋桨飞机利用货物隔板进行货物的分区及固定）。如果许可证里没有提到网绳的存在，那么在检查中如果发现网绳缺失，就不构成一项检查结果。 参考文件：附件6-I

# 附件C  预设问题和严重性分类

| 项目 代码 | 项 目 描 述 | 严重性 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 轻微（1类） | 严重（2类） | 重大（3类） |
| A01 | 总体 状况 | 飞行员（副驾驶）座椅上舱门的开锁/关锁装置没有或不可用 Door (un)locking mechanism at (Co)Pilot station N/A or U/S | 未向机组人员提供通知（如内话、密码锁）的方式 No means provided for crew notification | 监控舱门区域（门镜、摄像头）的方式没有或不可用（超出MEL限制） Means to monitor the door area not available or U/S (outside MEL limits |
|  |  | 从任一飞行员坐椅皆无监控舱门外侧区域的方式（但是为飞行关键阶段制定了备用操作程序） Means to monitor the door area not available from either pilot’s station (but alternative operational procedures established for the critical phases of the flight | 从任一飞行员坐席皆无监控舱门区域的方式（并且没有制定备用操作程序） Means to monitor the door area not available from either pilot’s station (and no alternative operational procedures established | 驾驶舱没有门锁或不可用（超出MEL限制） Cockpit door lock N/A or U/S (outside MEL limits) |
|  |  |  | 驾驶舱的装置严重影响飞行员视野 Cockpit installations significantly decreasing pilots vision | 飞行机组人员数量不足 Insufficient number of flight crew members |
|  |  |  | 可用的驾驶舱标记和/或标牌丢失或不准确 Operational flight deck markings and/or placards missing or incorrect | 飞行机组成员未遵守飞行时间和值勤时间的规定 Flight Crew member not in compliance with the flight and duty time rules |
|  |  |  | 驾驶舱中裸露在外的电缆/电线未被发现 Inadvertently exposed electrical cables/wires in the cockpit | 飞行期间内部设备和/或其他物品未正确固定或存放 Interior equipment and/or other object(s) not correctly secured or stowed during flight |
|  |  |  |  | （客机）未安装加固舱门 Reinforced cockpit door not installed (on passenger flights) |
|  |  |  |  | 警告面板上的灯光不可用（超出MEL的限制） Lights U/s in warning panel (outside MEL limits) |
|  |  |  |  | 风挡玻璃雨刷/清洗系统/干燥系统未安装或不起作用并且在降雨时这些设备的工作性能（超出MEL的限制） Windshield wipers/cleaning/drying system not installed or inoperative and their usage required due to precipitation (outside MEL limits) |
|  |  |  |  | 超出AMM限制的挡风玻璃脱层 Windshield delamination outside AMM limits |
| A02 | 应急 出口 |  |  | 应急出口通道受阻 Access to emergency exit impeded |
|  |  |  |  | 应急出口不可用 Emergency exits U/S |
|  |  |  |  | （如适用）驾驶舱的逃生设施（绳索、舱口、保护带）没有或不可用（超出MEL的限制） If applicable, flight deck escape facilities (ropes, hatches, harnesses) not available or unserviceable (outside MEL) |
| A03 | 运行设备和要求 |  | 首个适航证在2012年3月1日及以后签发的飞机未安装ACAS II 7.1版本 Aircraft with first CoA issued on or after 1 March 2012 not equipped with ACAS II, software version 7.1 | 所安装的必需设备虽安装但并未明确显示机组人员在飞机运行过程中使用 Required equipment installed but clearly not being used during operation by crew |
|  |  |  |  | 未安装ACAS II或ACAS II不可用(超出MEL 限制) ACAS II N/A or U/S (outside MEL limits) |
|  |  |  |  | 未安装具有前视地形避让功能的GPWS或者虽安装但不可用（超出MEL的限制） GPWS with forward looking terrain avoidance function not installed or unserviceable (outside MEL limits) |
|  |  |  |  | 所需的导航设备未安装或不适用（超出MEL的限制） Required navigation equipment N/A or U/S (outside MEL limits) |
|  |  |  |  | 驾驶舱语音记录仪（CVR）失效（超出MEL限制） Cockpit Voice Recorder inoperative (outside MEL limits) |
| A04 | 手册 |  | 有关飞行运行的手册未携带上机或不完整 No or incomplete parts of the Operations Manual pertaining to flight operations on board | 机上的性能和限制数据缺失或不完整 No or incomplete performance and limitations data on board |
|  |  |  | 运行手册中没有关于飞行时间、执勤时间和休息时间限制的规定 No rules on flight time, flight duty and rest time limitations in the Operations manual | 运行手册未就紧急情况下的处理方式给予信息和指导 No information and instructions in Operations Manual on the actions to be taken in the event of an emergency |
|  |  |  | 运行手册未更新 Operations manual not up to date | 运行手册以所有机组成员不能理解的语言发布 Operations Manual published in a language not understood by any of the flight crew members |
|  |  |  | 运行手册并非运营人发布 Operations manual not issued by the operator |  |
|  |  |  | 至少一名机组成员不能理解的语言发布 Operations Manual published in a language not understood by a member of the flight crew |  |
| A05 | 检查单 |  | 检查单与运行手册中的检查单内容不一致 Checklists do not conform with the checklist details in the operations manual | 机长和副驾驶所用的检查单版本不同 Different versions of checklists used by captain and co-pilot |
|  |  |  | 手册中缺少检查单的详细内容 No checklist details in the operations manuals | 无正常和紧急情况下可用的检查单 No normal and emergency checklists available |
|  |  |  | 所有相关的飞行机组人员不能快速的获取正常和应急检查单 Normal and emergency checklists not readily accessible to all relevant flight crew members |  |
|  |  |  | 检查单并未涵盖飞行各个阶段 Checklists not covering all flight phases |  |
| A06 | 无线电导航图 |  | 导航数据库过期（在MEL的限制内） Navigation database out of date (within MEL limits) | 导航数据库过期（超出MEL的限制） Navigation database out of date (outside MEL limits) |
|  |  |  | 所需航图过期（导航数据库更新） Required en-route charts out of date (navigation database up to date） | 所需航图和导航数据库过期 Required en-route charts and navigation database out of date |
|  |  |  | 几套所需的仪表图在驾驶舱内，但其中一套（未使用）过期 Several sets of required instrument charts available in the flight deck, of which one (not in use)is out of date | 所需仪表图（航路除外）过期 Required instrument charts (except en-route) out of date |
|  |  |  |  | 所需航图未携带上机 Required instrument charts not on board |
| A07 | 最低设备清单 |  | MEL未反映出飞机构型或运行规范 MEL does not reflect aircraft configuration or the operations specifications | MEL缺少（M）和/或（O）程序，保留故障需要此程序 MEL lacking (M) and/or (O) procedures when required (with deferred defect requiring such procedure) |
|  |  |  | MEL缺少（M）和/或（O）程序，保留故障不需要此程序 MEL lacking (M) and/or (O) procedures when required (no deferred defect requiring such procedure) | MEL比MMEL的限制性要低（因限制更低导致保留故障受到影响） MEL less restrictive than the MMEL (with deferred defects affected by the lower restrictions) |
|  |  |  | MEL比MMEL的限制性要低（更低的限制并未导致保留故障受到影响） MEL less restrictive than the MMEL (without deferred defects affected by the lower restrictions) | 某些MEL项目没有客户化（有故障保留的项目涉及） Some MEL items not fully customised (with defects affecting those items) |
|  |  |  | MEL不可用（无保留故障） MEL not available (no deferred defects) | MEL不可用（有保留故障） MEL not available (with deferred defects) |
|  |  |  | 某些MEL项目没有客户化（但是没有故障保留的项目涉及到这些项目） Some MEL items not fully customised (but no defects affecting those items |  |
|  |  |  | 用MMEL替代MEL MMEL instead of MEL |  |
| A08 | 运行合格证和运行规范或等效文件 | 有效的AOC和/或运行规范已签发但在航班运行时未带上飞机 A valid AOC and/or operations specifications for the flights performed was issued but not carried on board at the time of the inspection | 运行规范的格式内容不符合附件6的规定 information in the operations specifications not in accordance with Annex 6 | 实际运行不符合运行规范要求 Commercial Air Transport operations not in accordance with the operations specifications |
|  |  |  | AOC中的信息不正确 Information in AOC incorrect | AOC已失效 Commercial Air Transport operations without a valid AOC |
|  |  |  | 没有英语版本 No English translation | 机上没有AOC原件或复印件和/或运行规范的原件和复印件，或机组未能出示 No original nor copy of the AOC, and/or of the operations specifications on board or cannot be shown by the crew |
| A09 | 国籍登记证 | 登记证的表格与附件7不一致 CofR format not in accordance with Annex 7 | 没有有效登记证或机组未能出示登记证 No valid CofR or cannot be shown by crew |  |
|  |  | 没有英语版本 No English translation |  |  |
|  |  | 未安装防火标识牌 No fireproof identification plate |  |  |
|  |  | 登记证上的信息与标识牌的内容不匹配 Mismatch of data on CofR and identification plate |  |  |
|  |  | 登记证虽有效但未携带到飞机上 A valid CofR was issued but not carried on board. |  |  |
| A10 | 无线电执照 | 无线电执照上的信息不正确 Incorrect information on the Radio Station Licence | 没有签发的无线电执照 No Radio Station Licence issued |  |
|  |  | 无线电执照过期 Radio Station Licence on board expired |  |  |
|  |  | 无线电执照已签发但检查时未带上飞机 A valid Radio Station Licence was issued but not carried on board at the time of the inspection |  |  |
| A11 | 适航证 | 适航证的格式不符合附件8的要求 Format of CofA not in accordance with Annex 8 requirements | 没有英语版本 No English translation | 没有适航证或者飞机登记国未认定适航证有效 CofA not issued nor rendered valid by the State of registry |
|  |  | 机上未携带有效的适航证 A valid CofA was issued but not carried on board at the time of the inspection. |  | 未经审定同意就签署适航证 Endorsed CofA without permission of the State of inspection |
| A12 | 飞行准备 | 地面没有保存运行飞行计划 No copy of the operational flight plan retained on the ground | ATC飞行计划不正确 ATC Flight plan incorrect | 机载燃油少于ICAO要求的最小值 Fuel on board less than minimum ICAO requirements |
|  |  | 运行飞行计划没有机长签字 Operational flight plan not signed by the PIC | 运行飞行计划的内容和使用与运行手册不一致 Content and use of the Operational Flight plan not in accordance with the operations manual | 飞行机组不了解起飞机场、目的地机场和备降机场的相关航行通告 Flight crew unaware of the applicable departure, destination or alternate airports NOTAMs. |
|  |  | 当运行手册有规定时未执行燃油消耗监控 No fuel consumption monitoring performed when required by the OPS Manual | 所用的运行飞行计划不正确 Incorrect Operational Flight Plan | 没有可用天气预报表明目的地机场或目的地备降机场的状况达到或高于最低标准 No weather forecast available indicating that the destination or destination alternate aerodrome conditions are at or above minima |
|  |  |  |  | 性能或者燃油计算对飞行不可用或者明显不正确 Performance and/or fuel calculation not available or significantly incorrect for the flight |
|  |  |  |  | 在进行ETOPS运行时，选择的航路备降场不可用 Required en-route alternate(s) (ETOPS) not available |
|  |  |  |  | 起飞机场天气实况和目的机场天气预报未在离港前确认 Actual weather and weather forecast not checked before departure |
|  |  |  |  | 在已知结冰状态下未经合适审定和/或没有设备的情况下飞行 Flight operated in known icing conditions without suitable certification and/or equipment |
|  |  |  |  | 在地面结冰的情况下，机组或地面工作人员没有进行结冰检查 No icing inspection performed by crew or ground staff with ground icing conditions |
|  |  |  |  | 没有备降机场或选择的备降机场不合适 No or unsuitable alternate(s) airports selected |
|  |  |  |  | 没有可用的运行飞行计划 No Operational Flight Plan |
|  |  |  |  | 在ETOPS运行时选择的航路备降机场天气低于最低标准 Weather on required en-route alternate(s) below ETOPS minima |
|  |  | 已进行起飞前检查，但机长未确认飞机的维修放行单 Pre-flight inspection performed but the pilot in command did not certify that he is satisfied that the aircraft is airworthy | 在起飞前检查之前机长就已确认飞机的维修放行单 Pilot in command certified that he is satisfied that the aircraft is airworthy before the pre-flight inspection was performed | 未进行起飞前检查 Pre-flight inspection not performed |
|  |  |  | 已进行起飞前检查但未记录明显故障信息 Pre-flight inspection performed but without recording significant defects |  |
| A13 | 载重平衡单 |  | 在飞机限度内不准确的重量和/或者配平计算，但对性能计算无影响 Incorrect mass and/or balance calculations, within a/c limits, and having no effect on the performance calculations. | 在飞机限度内不准确的重量和/或者配平计算，并且对性能计算有影响 Incorrect mass and/or balance calculations, within a/c limits, but affecting the performance calculations |
|  |  |  | 数据不足导致机组人员无法检测重量和配平计算 Insufficient data to enable the crew to check the Mass & balance calculations | 重量和配平超出运行限制 Mass & balance outside operational limits |
|  |  |  | 装载舱单不能反映实际业载分布，但是未超出飞机限制 Load sheet does not reflect actual load distribution but within A/C limits | 重量和配平计算未完成 No mass and balance calculations performed |
|  |  |  |  | 机上的载重平衡表填写不完整 No completed mass and balance sheet on board |
| A14 | 手提灭火器 |  | 便携式灭火器（HFE）不在指定位置 HFE not at indicated location | HFE是空的、不可用或没有（超出MEL限制） HFE empty, unserviceable or missing (outside MEL limits) |
|  |  |  | HFE未标记相关使用说明 HFE not marked with the appropriate operating instructions | HFE不可用 HFE not accessible |
| A15 | 救生衣/漂浮设备 |  | 需要进行此类飞行时，救生衣/漂浮装置不易取用 Life jackets/flotation devices not easily accessible when required for the type of flight | 需要进行此类飞行时，可用的救生衣/漂浮装置数量不足 Insufficient number of life jackets/flotation devices available and required for the type of flight |
| A16 | 安全带 |  | 飞行员安全带没加入自动收紧装置 Pilot harness does not incorporate an automatic restraining device | 每个飞行员座位没有安全带或安全带不能使用 （超出MEL限制） No or unserviceable safety harness for each pilot seat (outside MEL limits) |
|  |  |  | 飞行机组的座位而不是飞行员座位没有安全带或安全带不能使用 No or unserviceable safety harness for a flight crew seat other than the pilot seats | 安全带缺失 No or unserviceable safety harness for each pilot seat (outside MEL limits) |
| A17 | 供氧设备 |  |  | 供氧设备未准备好且不符合飞行类型 Oxygen equipment not readily accessible and required for the type of flight |
|  |  |  |  | 可用的快速佩戴式氧气面罩数量不足 Insufficient number of serviceable quick donning masks available |
|  |  |  |  | 氧气和/或可用的氧气面具不充足 Insufficient oxygen and/or serviceable oxygen masks |
|  |  |  |  | 氧气系统不可用 Unserviceable oxygen system |
| A18 | 应急手电筒 | 夜间运行时只为在座飞行员而没有为其他的机组人员配备可用的电筒 Serviceable electric torch for both pilots but not for other flight crew members during night operation |  | 夜间飞行时可用电筒的数量不够全体飞行员使用 Electric torches not readily available during night operation |
|  |  |  |  | 在夜间飞行时电筒不便于使用 Insufficient number of serviceable electric torches for all pilots during night operation |
| A19 | 机组人员执照 | 已颁发有效且相应的飞行执照但在检查时未携带上机 A valid and appropriate Flight crew licence was issued but not carried on board at the time of the inspection. | 格式和/或内容不符合ICAO标准（执照、体检合格证） Form and/or content not in compliance with ICAO standard (licence, medical certificate) | 无机组成员持有有效的R/T执照/等级 No crewmember holds a valid R/T licence/rating |
|  |  |  | 没有声明说明与ICAO标准的差异 No declaration of licence differences compared to ICAO standards | 飞行机组成员的执照没有相应的型别等级 No appropriate type rating on flight crew member’s licence |
|  |  |  | ELP（英语能力证明）签注过期 ELP endorsment expired | 签注英语语言能力和/或英语语言能力未达到规定，低于四级（但颁照国向ICAO提交符合性文件） No endorsement of the required English language proficiency and / or level lower than Level 4 (whilst the licensing State filed compliance to ICAO) |
|  |  |  | 签注英语语言能力和/或英语语言能力未达到规定，低于四级（但纠正措施计划已由颁照国提交给ICAO） No endorsement of the required English language proficiency and / or level lower than Level 4 (but corrective action plan filed by the licensing State to ICAO). | 签注英语语言能力和/或英语语言能力未达到规定，低于四级（但纠正措施计划未由颁照国提交给ICAO） No endorsement of the required English language proficiency and / or level lower than Level 4 (and no corrective action plan filed by the licensing State to ICAO). |
|  |  |  | 没有国际民航组织规定的执照相关项目的英语译文 No English translation of ICAO required items of the licence | 两名飞行员年龄均大于６０岁 Both pilots older than 60 years |
|  |  |  | 未提及国际民航组织体检合格证等级 No mention of ICAO medical class | 机组人员没有相应的执照 Flight crew member without appropriate licence |
|  |  |  | 注册国颁发的执照的有效性不适用 No proper validation issued by the State of registry | 体检合格证在行使执照特许权期间失效 Medical certificate invalid for the privileges being exercised |
|  |  |  | 矫正眼镜不可用（对于多飞行员的运行） Spare correcting spectacles not available (for multi-pilot operations) | 在有需要时，没有可用的矫正眼镜 No correcting lenses available when required |
|  |  |  |  | 单机组运行的机长年龄超过60岁 PIC over 60 in single pilot operations |
|  |  |  |  | 多机组运行的机长年龄超过65岁 PIC over 65 in multipilot operations |
|  |  |  |  | 没有可用的备用矫正眼镜（单机组运行） Spare correcting spectacles not available (for single pilot operations) |
| A20 | 飞行记录本或等效文件 | 不一致的数据录入飞行记录本 Inconsistent data entered into the Journey Log Book | 飞行记录本或一般报告不在飞机上 Flight details not recorded in a journey logbook or General Declaration |  |
|  |  | 飞行详情没有记录在飞行记录本或一般报告中 Journey logbook or General Declaration not on board |  |  |
| B01 | 内部整体状况 | 墙面损坏 Damaged wall panels | 设备安装明显不符合附件8，第三部分A/B，第四章的规定 Equipment installations obviously not in compliance with Annex 8, Part IIIA/B,Chapter 4 | 洗手间未配备烟雾探测系统 Lavatory(s) not equipped with smoke detection system |
|  |  |  | 座舱内部布局明显不符合规定设计说明书中关于易燃材料的条款 Cabin interior layout obviously not furnished in accordance with certified design specifications concerning flammable materials | 垃圾箱未配备内置灭火系统 Disposal receptacles not equipped with a built-in fire extinguisher system |
|  |  |  |  | 在飞行期间机组人员随身行李未固定 Crew carry-on baggage not adequately and securely stowed during flight |
|  |  |  |  | 客舱/厨房内物品过重或松动 Loose or heavy objects in the cabin/galleys |
|  |  |  |  | 客舱设备未正确固定 Cabin equipment not properly secured |
|  |  |  |  | 行李存放不当或洗手间内物品松脱 Inappropriate stowage of luggage or loose articles in the toilets |
|  |  |  |  | 洗手间内烟雾探测装置阻塞 Lavatory smoke detection system obstructed |
|  |  |  |  | 洗手间故障（未被标出或不符合MEL的要求） Lavatory inoperative (not placarded as such and not confirmed with MEL restrictions if any) |
|  |  |  |  | 厨房/洗手间废物箱的盖板故障（超出MEL限定范围） Galley/lavatory waste receptacle access door cover inoperative (outside MEL limits) |
|  |  |  |  | 服务手推车刹车明显故障 Obviously defective brakes of service cart(s) |
|  |  |  |  | 外盖损坏/缺失而暴露出锋利边缘和/或电缆和电线 Covers damaged/missing exposing sharp edges and/or cables and wires |
|  |  |  |  | 舱顶板储藏槽不可用（无法识别） Overhead bins unserviceable (and not identified as such) |
| B02 | 客舱乘务员座椅/机组休息 区 | 安全带或锁扣磨损或损坏 Strap or buckle worn or damaged | 客舱机组人员座椅未配备肩带（仅配备了安全带） Cabin Crew seat(s) not equipped with safety harness (only seat belt) | 客舱机组座椅不可用（超出MEL的限制） Cabin Crew life jackets (when required) not easily accessible |
|  |  |  | （当需要时）客舱机组人员救生衣不易拿到 Cabin Crew life jackets (when required) not easily accessible | 客舱机组安全带缺失或不可用 Cabin Crew seat(s) unserviceable (outside MEL limits) |
|  |  |  |  | 客舱机组座椅明显安装错误（超过飞机纵轴的15度） Cabin Crew seat(s) obviously not installed correctly (more than 15 degrees from the longitudinal axis) |
|  |  |  |  | 客舱机组座椅安装错误 Cabin Crew seats not correctly located |
|  |  |  |  | 通信设备不可用（超出MEL的限制） Communication equipment unserviceable (outside MEL limits) |
| B03 | 急救/应急医疗箱 | 医疗用品未放在指定位置 Medical supplies not at the indicated location | 医疗用品标识不清 Medical supplies not identified as such | 医疗箱中的物品过期 Contents of the medical kit past expiration date |
|  |  | 急救箱/卫生防疫包中的物品过期 Contents of the first aid kit/universal precaution kit past expiration date |  | 飞行途中医疗用品缺失或不可用 Medical supplies not available or not accessible during flight |
| B04 | 手提灭火器 |  | 手提灭火器未装配在指定位置 HFE not at indicated location | 手提灭火器耗尽（灭火剂）、不可用或缺失（超出MEL的限定） HFE not marked with the appropriate operating instructions |
|  |  |  | 手提灭火器的相关操作说明未被标出 HFE empty, unserviceable or missing (outside MEL limits) | 手提灭火器未安放在正确位置 HFE not correctly secured |
|  |  |  |  | 手提灭火器不易取用 HFE not readily accessible |
| B05 | 救生衣/漂浮设备 |  | 救生衣/ 漂浮设备使用不便或与飞行种类要求不符 Life jackets / Flotation devices not easily accessible and required for the type of flight | 符合飞行种类的可用救生衣或设备数量不足 Insufficient number of serviceable Life jackets / Flotation devices available and required for the type of flight |
| B06 | 座椅数量，座椅安全带和座椅状况 | 乘客座椅破旧 Passenger seats in poor condition | 安全带或锁扣破损或损坏 Strap or buckle worn out or damaged | 机上未配备加长安全带 No extension belts available on board and required |
|  |  |  |  | 机上可用乘客座椅安全带数量不足 No serviceable seat belt available for each passenger on board |
|  |  |  |  | 座椅不可用且未标明 Seat(s) unserviceable and not identified as such |
|  |  |  |  | 儿童床位无绑带 Baby berth(s) used without restraining belts |
|  |  |  |  | 飞机上的座位数量不足Insufficient seats for all passengers on board |
|  |  |  |  | 飞机上婴儿座位/床位安装不合格 Seat(s)/baby berth(s) not certified to be installed on board of aircraft |
| B07 | 应急出口，照明设备和应急手电筒 | 应急出口标识盖/罩缺失或破损 Emergency exit sign(s) lens/cover missing or broken | 夜间飞行时客舱机组成员可用电筒数量不足 Insufficient number of serviceable torches for each cabin crew member during night operations | 应急出口标志故障（超出MEL规定范围） Emergency exit sign(s) out of order (outside MEL limits). |
|  |  | 小桌板扣在受到减速力或冲击力影响时无法扣住小桌板使其保持锁死状态（对于不靠近应急出口的座椅来说） | 夜间飞行时，一些客舱的电筒不可用 Electric torches not readily accessible for some of the cabin crew during night operations | 应急出口目视指示设备不可用（超出MEL限制） System for visually indicating the escape path(s) unserviceable (outside MEL limits). |
|  |  |  | 应急出口正确操作指示未被标明 Emergency exit(s) not marked with the appropriate operating instructions | 夜间飞行时客舱机组人员手电筒不易取用 Cabin crew members’electric torches not readily accessible during night operations |
|  |  |  | 没有应急路径照明手段 No means for illuminating the escape paths | 应急出口，照明设备，标志不可用（超出MEL限定范围） Emergency exit(s),lighting and marking unserviceable (outside MEL) |
|  |  |  | 应急通道附近的墙面损坏可能阻挡出口 Damaged wall panel in the vicinity of emergency exit possibly obstructing the exit | 应急出口不可用的情况下，机上乘客数量超过规定的最大值 Number of passengers on board exceeds the maximum allowed in case of unserviceable emergency exit(s) |
|  |  |  |  | 地板/地毯质量不好而影响快速撤离 Floor/carpet in poor condition affecting the rapid evacuation |
|  |  |  |  | 小桌板锁扣可朝疏散方向打开（不是嵌壁式的或特殊一端上锁） Tray table latches can be opened in the direction of evacuation (not recessed or special one-way lock) |
|  |  |  |  | 应急出口的通道被行李或物品堵塞 Access to emergency exits impeded by baggage or cargo |
|  |  |  |  | 乘务员的座椅没有自动弹回而阻塞应急出口通道 Cabin crew seat does not retract automatically impeding the access to emergency exit |
|  |  |  |  | 应急出口的通道被座椅堵塞 （座椅垫过大） Access to emergency exits impeded by seats (total rows) |
|  |  |  |  | 特制小桌板锁扣可朝飞机疏散的方向打开 Access to emergency exits impeded by seats (oversized seat cushions) |
| B08 | 滑梯/救生筏（如适用）应急定位发射器 |  | 未按要求配备长距离水上飞行时用于发射烟火救生信号的设备 No equipment for making the pyrotechnical distress signals when required for long-range over water flights | 可用的滑梯/紧急滑梯救生筏数量不足 Insufficient number of serviceable slides/slide rafts |
|  |  |  |  | 长航线跨水运行可用的救生筏数量不足 Insufficient number of serviceable rafts and required for long-range over water flights |
|  |  |  |  | 可用ELT数量不足（超出MEL限制范围） Insufficient number of compliant ELTs (outside MEL limits) |
|  |  |  |  | 没有能够同时在406 MHz 和121.5 MHz之间转换的ELT No ELT capable of simultaneously transmitting on 406 MHz and 121.5 MHZ |
|  |  |  |  | 便携式ELT不在指定位置 Portable ELT not at indicated location |
| B09 | 供氧设备 |  | 便携式氧气设备不在指定位置 Portable breathing equipment not at indicated location | 飞机未配备自动展开供氧系统（单机适航证在1998年11月9日或之后颁发并且计划飞行高度超过FL250） Aeroplane not equipped with an automatic deployable oxygen system (individual CofA issued on or after 9 November 1998) and flight planned above FL 250 |
|  |  |  | 供氧设备与航班类型不符 Oxygen equipment not readily accessible and required for the type of flight | 所需自动供氧组件数量不足-单机适航证在1998年11月9日或之后颁发 Insufficient number of required serviceable automatic deployable oxygen dispensing units - individual CofA issued on or after 9 November 1998 (outside MEL limits) |
|  |  |  | 供氧设备使用说明标记不够充分 Oxygen equipment not adequately marked with its operating instructions | 航班类型要求的供氧量和/或可用的氧气面罩不足 Insufficient oxygen quantity and/or serviceable oxygen masks required for the type of flight |
|  |  |  |  | 自动供氧分配系统不可用（损坏/粘连，掉出舱顶板）超出MEL限制范围 Automatic oxygen deploying system unserviceable (damaged/ taped drop-out panels) outside MEL limits |
|  |  |  |  | 氧气分配设备不可用（低压，明显过期，损坏）未证实符合航班类型需要 Oxygen dispensing equipment unserviceable (low pressure, clearly overdue, damaged) and not identified as such and required for the type of flight |
|  |  |  |  | 氧气瓶未正确固定 Oxygen bottles not correctly secured |
| B10 | 安全须 知 | 没有充足的安全须知卡来提供给飞机上的所有乘客 Insufficient Aircraft Briefing Cards for all passengers on board | 安全须知卡包含错误信息 Aircraft briefing cards contain inaccurate information | 洗手间内“请回座椅”信号灯故障（超出MEL限定） 'Return to Seat' signs in lavatory unserviceable (outside MEL limits) |
|  |  | 安全须知卡破损或模糊 Aircraft briefing cards in poor condition | “扣紧安全带”信号灯故障 'Fasten seat belt' sign(s) unserviceable | 飞机上没有安全须知卡或其他传递给乘客安全指示的途径 No aircraft briefing cards on board and no other means to convey safety instructions to the passengers |
|  |  |  |  | 安全须知卡内容与机型和/或飞机结构不匹配 Aircraft briefing cards not for the correct aircraft type and/or configuration |
| B11 | 客舱机组成员 |  | 客舱乘务员不熟悉客舱应急程序 Cabin crew member(s) not familiar with the cabin emergency procedures | 客舱乘务员数量不足 Insufficient number of cabin crew members |
|  |  |  | 客舱乘务员不熟悉应急设备的位置和/或操作 Cabin crew not familiar with the location and/or operation of emergency equipment | 当给载有乘客的飞机加油时，相关乘务员未在指定位置 Qualified personnel not at their required positions when refuelling with passengers on board |
|  |  |  |  | 当给载有乘客的飞机加油时未与地面人员建立双向通信 No two-way communication established with the ground crew during refuelling with passengers on board |
|  |  |  |  | 客舱乘务员未遵守飞行值勤期规定 Cabin Crew member not in compliance with the flight and duty time rules |
| B12 | 旅客行李安全 |  |  | 过硬或过重的行李存放在开放式帽架上 Hard or heavy baggage stored in open hat-racks |
|  |  |  |  | 行李存放在不可用的舱顶板箱（尽管已标记为不可用） Baggage stowed in unserviceable overhead bins (although marked as unserviceable) |
|  |  |  |  | 尺寸过大的行李在客舱内的移动没有足够的安全保障 Oversized baggage transported in the cabin not adequately secured |
|  |  |  |  | 行李未被安全存放 Baggage not stowed securely |
|  |  |  |  | 舱顶板箱载重量超过标牌上的重量限制 Overhead bins loaded in excess of the placarded weight limitation |
| C01 | 外部总体状况 | 要求的标记和/或标牌不可用或难以辨认 Markings and/or placards required by the manufacturer not applied or unreadable | 破开点标记（如果使用）褪色或标记错误 Break-in point markings (if applied) faded or incorrectly marked | 夜间飞行飞机灯光失效（超出MEL限制） Aircraft lights unserviceable for night operations (outside MEL limits) |
|  |  | 机身过脏影响检查效果Aircraft very dirty affecting the ability to inspect it | 漆面受损导致复合材料暴露 Paint damage with exposed composite | 锁扣或铆钉松动或缺失超出SRM或AMM限制 Fasteners or rivets loose or missing outside SRM/AMM limits |
|  |  |  | 除防冰系统故障 Poor condition of de/anti-icing system | 静电放电器缺失或损坏超出MEL/AMM/CDL限制 Static discharger(s) missing or damaged outside MEL/AMM/ CDL limits |
|  |  |  | 未使用安全标记或使用的安全标记难以辨认 Safety markings not applied or unreadable | 天线缺失或损坏超出MEL/AMM/CDL限制 Antenna(s) missing or damaged outside AMM/MEL/CDL limits |
|  |  | 轻度腐蚀 Minor corrosion | 重度腐蚀 Significant corrosion | 运营人无意对飞机进行除冰作业 No intentions to request appropriate de-icing treatment |
|  |  |  | 锁扣、铆钉松动或缺失超出限制，但是根据AMM或SRM允许放行，且未作评估或记录Fasteners/rivets loose or missing outside limits, but dispatch allowed according to AMM/SRM, and not assessed nor recorded | 在地面有冰的情况下没有进行相关的除防冰作业 No appropriate de/anti-icing treatment with ground icing conditions |
|  |  |  |  | 压力孔受损或受污 Pressure port(s) damaged or contaminated |
|  |  |  |  | 尾撬磨损超出AMM限制 Tail skid wear outside AMM limits |
| C02 | 舱门和舱口 |  | 舱门把手、手柄、观察板不齐平 Door handle(s), lever(s), access panel(s) not flush | 电缆破损或缺失（超出AMM限制） Bonding wires broken or missing (outside AMM limits) |
|  |  |  | 舱门的使用说明缺失或不明确 Door operation instructions missing or unclear | 货舱门锁检查镜失效且没有其他办法去验证锁定的位置 Cargo door lock inspection glasses blind and no other mean to verify locking position(s) |
|  |  |  |  | 舱门密封圈受损超出AMM/CDL限制 Door seal damaged outside AMM/CDL limits |
|  |  |  |  | 舱门不可用（超出AMM/CDL限制） Door(s) unserviceable outside AMM/CDL limits |
| C03 | 飞行操纵舵面 |  |  | 电缆破损或缺失（超出限制） Bonding wires broken or missing (outside limits) |
|  |  |  |  | 液压泄漏超出限制 Hydraulic leak outside limits |
|  |  |  |  | 静电放电器缺失（超出MEL/AMM/CDL限制） Static discharger(s) missing (outside MEL/AMM/CDL limits) |
|  |  |  |  | 飞行控制失效 Flight controls unserviceable |
|  |  |  |  | 锁扣或铆钉松散或缺失超出AMM/SRM限制 Fasteners or rivets loose or missing outside AMM/SRM limits |
| C04 | 机轮，轮胎和刹车 | 刹车磨损指针缺失（至少保留一个） Brake wear indicator pin(s) missing (at least one pin remaining) | 刹车磨损超出限制但根据AMM准许放行（未评估也未记录） Brake worn beyond limits but dispatch allowed according to AMM (not assessed nor recorded) | 刹车失效且无记录 Brake(s) unserviceable and not recorded |
|  |  | 轮胎增压阀盖丢失 Tyre inflation valve(s) cap missing | 轮胎磨损超出限制但根据AMM准许放行（未评估也未记录） Tyre(s) worn beyond limits but dispatch allowed according to AMM (not assessed nor recorded) | 零件损坏或缺失超出限制（例如：螺锁扣、加热传感器） Damaged or missing parts outside limits (i.e. bolts, heat sensors) |
|  |  | 刹车组件引气阀盖丢失 Brake assembly bleed valve dust cap(s) missing |  | 液压刹车系统漏油（超出限制） Leaking hydraulic braking system(outside limits) |
|  |  |  |  | 轮胎气压明显超出限制 Tyre pressure obviously outside limits |
|  |  |  |  | 轮胎失效（磨损或损坏）且无记录 Tyre(s) unserviceable (worn or damaged) and not recorded |
|  |  |  |  | 轮缘受损超出限制 Rim damaged outside of limits |
| C05 | 起落架、橇/浮筒 | 要求的标记和/或标牌不可使用或难以辨认 Markings and/or placards required by the manufacture not applied or unreadable | 电缆管道磨损 Lines, hoses electrical wiring chafed | 水/碎片折流板损坏或丢失超出AMM/CDL限制 Water/debris deflectors damaged or missing outside AMM/CDL |
|  |  | 安全锁插销缺失或故障 Safety lock pin(s) missing or defective | 安全标记无法使用或难以辨认 Safety markings not applied or unreadable | 渗漏/泄漏超出限制 Seepage/leakage outside limits |
|  |  | 起落架过脏影响检查效果并且可能引发泄漏 Undercarriage dirty affecting the ability to inspect it and detect potential leakages | 重要标记被腐蚀 Significant signs of corrosion | 撑杆压力超限 Strut pressure outside limit |
|  |  | 起落架撑杆阀盖丢失 Gear strut valve cap(s) missing |  | 起落架减震支柱镜面长度超限 Nose landing gear wheel snubbers worn outside limits |
| C06 | 轮舱 | 轮舱受污影响检查效果 Wheel well dirty affecting the ability to inspect it | 铰链、作动筒明显缺少润滑 Obvious lack of lubrication of hinge(s),actuator(s) | 起落架舱门损坏超出SRM限度 Landing gear door(s) damaged outside SRM limits |
|  |  |  |  | 电缆破损或丢失（超出限度） Bonding wires broken or missing (outside limits) |
|  |  |  |  | 重要标记被腐蚀 Significant signs of corrosion |
|  |  |  |  | 起落架应急弹簧锁破损、失效 Landing gear emergency spring lock(s) broken/unserviceable |
|  |  |  |  | 渗漏/泄漏超出限度 Seepage/leakage outside limits |
| C07 | 发动机和吊架 | 要求的标记和/或标牌不可使用或难以辨认 Markings and/or placards required by the manufacturer not applied or unreadable | 安全标记不可用或难以辨认 Safety markings not applied or unreadable | 损坏（凹痕、划痕、裂纹）超出限度 Damage (dents, nicks, cracks) outside limits |
|  |  |  | 进气道和排气道区域受损严重 Significant damage in the intake and exhaust area | 进声线损坏超出AMM限度 Intake acoustic liners damaged outside AMM limits |
|  |  |  |  | 泄漏（油、燃油、液压）超出AMM限度 Leakage (oil, fuel, hydraulics) outside AMM limits |
|  |  |  |  | 面板/整流罩/保护罩/把手无法对准及无法齐平超出限度 Panels/fairings/cowlings/handles misaligned or not flush outside AMM limits |
|  |  |  |  | 螺丝/铆钉松动或缺失超出限度 Screws/rivets loose or missing, outside limits |
|  |  |  |  | 推力反推装置/阻流门未完全收上 Thrust reverser/blocker doors not fully stowed |
| C08 | 风扇叶片 |  |  | 风扇叶片损坏超出AMM限度 Fan blade(s) damaged beyond AMM limit |
| C09 | 螺旋桨、旋翼（主体/尾部） |  |  | 螺旋桨除冰系统失效（超出MEL/AMM限度） Propeller de-icing system unserviceable (outside MEL/AMM limits) |
|  |  |  |  | 螺旋桨损坏超出AMM限度 Propeller(s) damaged beyond AMM limits |
| C10 | 明显的维修 |  | 临时修理信息的缺失 No information about temporary repairs | 明显未按照AMM/SRM的规定进行修理 Repairs obviously not carried out in accordance with the applicable AMM/SRM |
|  |  |  | 先前的维修状况较差 Previous repair in poor condition |  |
| C11 | 明显未经维修的损伤 |  |  | 影响飞机适航性的结构性损坏 Structural damage affecting the airworthiness of the aircraft |
| C12 | 泄 漏 |  |  | 泄漏超出限制范围 Leakage outside limits |
|  |  |  |  | 维修门/维修面板，排水管被冰或其他碎片阻塞 Servicing doors/panels, drains blocked by ice or other debris |
| C13 | 维修放行，故障通告和修理 | 保留故障参考了错误的MEL/CDL Defect deferred with a wrong MEL/CDL reference | 维修操作未正当报告 Maintenance action not properly reported |  |
|  |  | 检查项目结束但未在故障保留清单中据实记录/持有项目清单 Item closed but not reported as such in the deferred defect list /hold item list | 保留故障在最后期限后结束 Deferred defect closed after the deadline | 保留故障在MEL修正间隔到期时开启 Deferred defect open while the MEL rectification interval has expired |
|  |  |  | 已知故障未报告/评估 Known defect(s) not reported/assessed | 技术记录本录入的条目未被机组成员理解 Technical logbook entry not understood by the flight crew members |
|  |  |  | 对于严重故障没有识别或监控的证据 No evidence of identification nor monitoring of significant defect | 故障予以保留但未（正确）采用规定的（M）（O）和/或MEL规定的其他程序 Defect deferred but without applying (correctly) the required (M), (O) and/or other procedures prescribed by the MEL. |
|  |  |  | ATLB中设定规定的修正间隔超出了MEL规定的修正间隔（但仍在MEL修正间隔范围内） Rectification interval set in the ATLB exceeding the rectification interval prescribed by the MEL (but still within the MEL rectification interval) | 要求的维修作业未执行或与适用的（MEL/AMM/SRM）说明不一致 Required maintenance action not performed or not in accordance with applicable (MEL/AMM/SRM) instructions. |
|  |  |  |  | 维修作业未由有资质的人员实施 Maintenance action not performed by appropriately qualified personnel. |
|  |  |  |  | 从事飞机维修的人员未使用适当的工具和/或技术数据 Maintenance personnel working on the aircraft without using appropriate tooling and/or technical data |
| D01 | 货舱总体状况 | 对安全性影响较小的轻微故障 Minor defects with limited effect on safety | 设备安装明显与附件8不一致 Equipment installations obviously not in compliance with Annex 8, Part IIIA/B, Chapter 4 | 货舱烟雾探测测试未达到或超出MEL限制 Cargo bay smoke detection test fail or outside MEL limits |
|  |  |  | 安全标志不可用或无法辨认 Safety markings not applied or unreadable | 释压口盖被挤压，损坏或丢失（超出AMM/MEL限制） Blow-out panels pushed, damaged or missing (outside AMM/MEL limits) |
|  |  |  |  | 面板损坏和/或处于限制范围之外 Damage to panelling and/or lining outside limits |
|  |  |  |  | 灭火系统失效且货舱仍在使用 Unserviceable fire extinguishing system and the affected cargo compartment is used |
|  |  |  |  | 地板锁失效超出MEL限制（装有货物） Floor locks unserviceable outside MEL limits (with cargo) |
|  |  |  |  | 屏障网缺失或不可用 No or unserviceable required barrier net |
|  |  |  |  | 无隔烟板/幕 No smoke barrier/curtain (if applicable) |
|  |  |  |  | 结构性或地面损害超出AMM/SRM限制 Structural or floor damage outside AMM/SRM limits |
|  |  |  |  | 货舱无灭火系统（如适用） Cargo compartment (s) not equipped with fire suppression systems |
|  |  |  |  | 货舱照明系统损坏超出AMM/MEL限定 Cargo compartment lighting damaged outside AMM/MEL limits |
| D02 | 机载货物的安全 | 捆绑、固定装置、货盘、锁具组件和/或集装箱轻微故障 Minor damage to lashing, tie-down equipment, pallets, lock assemblies and/or containers | 捆绑、固定装置、货盘、锁具组件和/或集装箱装备不全 Incomplete equipment like lashing, tie-down equipment, pallets, lock assemblies and/or containers | 货物区的划分使用不符合分区要求 Cargo Area not used in accordance with classification |
|  |  |  |  | 货物未被全方位安全绑缚并牢固固定 Cargo not correctly secured and restrained in all directions |
|  |  |  |  | 影响结构完整和预期作用的捆绑、固定装置、货盘、锁具组件和/或集装箱的重大故障 Major damage to lashing,tie-down equipment,pallets, lock assembliesand/or containers affectingthe structural integrity and their intended function |
|  |  |  |  | 划分网绳或保护网绳受损超出AMM限制 Dividing net or protection net damaged beyond AMM limits |
|  |  |  |  | 载荷分布/载荷超出规定限制 Load distribution/load limit (floor and/or height) exceeded |

# 附件D 发现问题通知书

NOTIFICATION OF FINDINGS

| Operator （运营人）: | Route from （进港航线）: | Scheduled/Nonscheduled Flight （定期/非定期）: |
| --- | --- | --- |
| State of the Operator （运营人所在国）: | Inbound Flight No. （进港航班号）: | Aircraft Configuration(Passenger/Cargo) （客机/货机）: |
| Date of Inspection （检查日期/年月日）: | Route to （离港航线）: | Registration Marks （飞机注册号）: |
| Local Time （检查时间）: | Outbound Flight No. （离港航班号）: | Captain （机长）: |
| Airport （检查地点）: | Aircraft Type （飞机型号）: | First Officer （副驾驶）: |

| 问题序号 No. | 检查项目编号Code. | 问题描述及参考资料 Findings | 问题类别 Cat.Minor/Significant/Major） | 整改要求 Corrective action taken |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

整改措施应当于30个日历日内提交

Corrective actions should be taken within 30 calendar days

采取的措施（Action taken by CAAC）对应项目打

| 1 | Inform action to Captain | 2 | Inform action to the CAA |
| --- | --- | --- | --- |
| 3 | Restriction on aircraft takeoff | 4 | Others:________________ |

检查单位: CAAC

监察员（1）签名：         Signature of Inspectors:

监察员（2）签名（如适用）：Signature of Inspectors:

日期Date (yyyy-mm-dd):

- 附件E 通知外国民航局模板

October 15, 2015

Mr./Miss:****

Title:

CAA:

Address:

Dear Mr./Miss:

We would like to inform you that Civil Aviation Administration of China (CAAC) performed a ramp inspection on XXX Airline B777-300ER aircraft, registration NXXXX, on October 10, 2015.The inspection report has revealed some significant and major findings.CAAC will be glad to share the data to you.The findings are as follows:

****

I thank you for your cooperation in the field of air transportation safety and inform you that additional ramp inspections may occur when aircraft of XXX airline land in the territory of China.Should you require any additional information on this matter, do not hesitate to contact us.

Yours sincerely

Mr./Miss

Director

CAAC

# 附件F  外航检查用语样例

一、飞行运行篇

1、Sir, could you show us the licence and Medical Certificate？Please show us the ICAO English Proficiency and the expiration date.

1、请出示驾照、体检合格证和ICAO英语等级及有效期。

2、Since there is limitation in the Medical Certificate that you should equip the spare correcting lenses. Could you show it?

2、你可以出示体检合格证中要求配备的备份纠正视力的眼镜吗？

3、According to Chicago Convention Article 29, the flight crew shall carry relevant licences. If you could not show the licences, please demonstrate that you are qualified to fulfill your duty.

3、根据公约29条，机组执行国际航班任务时，应携带相关证照。你无法出示证照，请用其他方式证明你的资质。

4、Your licence is issued by XXX country other than the State of Registry of the aircraft. Could you show us the authorization of the state of Registry of the aircraft?

4、你持有的执照是由其他机构而非登记国颁发，请出示你的登记国出具的认可函。

5、Is your duty time in compliance with the requirements of OM?

5、你的值勤时间是否符合手册中对于飞行时间和休息时间的规定

二、航务篇

- Could you tell us the fuel on board?

可以告知飞机上的剩油吗？

2、Could you show us the Radio Navigation Charts required for the inbound and outbound flight and demonstrate their validity.

2、请出示本次航班运行使用的航图,并证明是最新有效的（确认航图是有效、完整的）；

3、Could you show us the flight documents (including CFPL ,Weather and NOTAM)? (The CFPL inbound should have been signed by the PIC.)

3、请提供进出港航班的飞行计划、天气和航行通告（确认机组是否签字）。

4、Is there anything special happened in the departure, destination and alternate airport that may affect the operation? For example, are you aware that the alternate may be closed when you plan to divert？

4、在起飞、目的地和备降场有什么影响运行的事情发生吗？(检查签派准备的航行通告和机组对资料的熟悉)

5、Is there any MEL item remark? Could you tell us the limitation of the operation in the CFPL?

5、飞机有影响运行的保留故障吗？在飞行计划中有无体现运行限制。

6、Are there any requirements of fuel management in the OPS? Have you followed the requirements?

6、在公司的运行手册中有对油量进行监控的要求吗？你们是否按要求进行了油量监控？

7、Since it is raining/snowing /the runway is shortened due to maintenance, have you checked the performance information of the aircraft?

7、今天下雨/下雪/跑道因维修缩短距离，你检查了飞机的性能限制信息了吗？

8、Could you show me the RVSM conversion table? And do you think it’s easy for you to fetch and use during flight?

8、可以出示RVSM高度层转换表吗？在运行过程中是方便取用的吗？

9、It’s RNAV operation in this airport/RVSM operation within China, could you show us the designator contained in the ATC flight plan ?

10、机场为PBN运行/中国空域为RVSM运行，你可以告知领航计划报中填写的项目吗？(核查PBN/RVSM能力与实际填写是否一致)

10、Could you show us the weight and balance sheet. There was additional fuel loaded. It should be included on the weight and balance sheet.

10、请提供进出港的载重平衡单。机组额外增加了燃油，应该在载重平衡单中进行体现和修正。

11、You have 3 additional flight crew. Have you check the correction of the DOW?

11、今天有多出3名机组人员，你是否检查了对干操作重量的修正？

三、客舱篇

1、Hello, I’m cabin inspector. I will do a routine inspection today. Could you tell me who is the purser?

1、你们好。我是客舱监察员，我今天将做一次例行检查。请问谁是乘务长？

2、Could you tell me how many attendants there are in this flight?

2、请告诉我本次航班共有多少名乘务员。

3、Please show me your licences.

3、请出示你们的证件。

4、Please tell me where the portable fire extinguishers/Life jackets/First aid kit are/is located. Have you checked whether they are available?

4、请指出你们的手提灭火器/救生衣/急救箱的位置。你们是否检查过这些设备的可用性？

5、Please tell me how to use the portable fire extinguisher?

5、请告诉我怎样使用手提灭火器？

6、Please open the Emergency lighting. I’d like to check whether it’s working.

6、请打开应急照明设备。我想确认是否都能正常工作。

7、How many life rafts are there in this plane? Have you checked whether they are available?

7、请问你们飞机上有多少个救生筏，你们是否检查过这些救生筏的可用性？

8、How many Oxygen supply are there in this plane? Have you checked whether they are available?

8、请问你们飞机上有多少个便携式呼吸设备，你们是否检查过这些呼吸设备的可用性？

9、There is no passenger briefing cards in your plane. Please introduce how to deliver the related information to passengers.

9、你们的飞机上没有安全须知卡，请介绍下你们怎么把相关信息传达给乘客的。

四、适航篇 (感觉有些例句是检查单的内容)

1、Who is the maintenance agent of this flight?

1、该航班的维修代理是谁？

2、Who is responsible for the aircraft’s maintenance release ?

2、谁负责该航班的维修放行？

3、Please show me your mechanic licence with aircraft type rating.

3、请出示带有机型的维修执照。

4、Please show me the technical journey log book or equivalent.

4、请出示飞机技术记录本。

5、Is there any deferred defect of this aircraft?

5、本架飞机有保留故障么？

6、Could you show me the MEL associated paragraph to this deferred defect？

6、请把此保留故障对应的MEL清单对应章节出示给我。

7、Have you completed maintenance job for this deferred defect?

7、你是否完成保留故障所要求的维修工作了么？

8、Please show me your walk around checklist?

8、请出示绕机检查的检查单。

9、Did you have some ETOPS check item for this flight?

9、此次航班有ETOPS检查项目么？

10、Could you show me the damage tolerance standard for this airframe damage.

10、请提供机身损伤的允许标准。

11、Please show me the brand of oil used in your aircraft.

11、请提供你们飞机使用的滑油牌号。

12、Could you please provide us with a sealing ring (a adjustable wrench, a round file, a triangle ,a wrench, a socket wrench ,a cutting pliers, a plastic hammer, some split cotters, some safety wire ,chamois leather, a piece of rag ,a bucket ,a washer, screwdriver, clamps ,pliers , flashlight)?

12、能提供我们一个密封圈（活动扳手，圆挫，三角挫，万用扳手，刻丝钳，橡胶锤，一些开口销，一些保险丝，鹿皮布，一块抹布，油桶，垫片，螺丝刀，夹子，钳子，手电筒）吗？

13、Please wipe the excess oil from the engine nacelles and landing gears. (监察员好像用不上)

13、请擦掉发动机吊舱和起落架上多余的滑油。

14、Please tighten the engine oil tank filler caps (Please check the oil cap. 这句怪怪的。)

14、请把发动机滑油加油口盖盖紧。

15、Please check there is no damage for fan blades and turbine blades. Please check there are no metal particles and visible damage on the engine tail pipe.

15、检查风扇叶片和涡轮叶片无损坏迹象。检查发动机尾喷管无金属颗粒和损坏。

16、Please check there is no damage and leakage for gear assembly structure attachment and up-assembly. (What do you think of the damage/leakage/crack? Please show me the standard.)

16、起落架组件结构，连接及上锁组件无损伤和渗漏现象。

17、I have found 3#tyre in the left MLG damaged seriously, please check it.

17、我发现左主起落架3号轮磨损严重，请检查。

18、Please arrange water servicing unit /a toilet servicing unit /an air conditioning unit /a deicing unit /a ladder for your airplane. (监察员好像用不上)

18、 请为你们飞机安排一辆加水车/放污车/空调车/除冰车/工作梯。

19、How much fuel have you filled in actually? We have put 60000 littler in total. (Please show me the refueling record.)

19、你实际给飞机加了多少油？总共60000公升燃油。

20、Visually check cargo compartment and door jambs, make sure that no liquid flows out or corrosion due to chemical produce, sea food and poultry etc. (感觉是检查单的内容)

20、目视检查货舱及厂门框，没有因化学制品，海鲜，家禽等货物而造成液体溢出或腐蚀现象。

五、介绍篇

1、Excuse me, I’d like to introduce myself first. I’m an inspector from CAAC.

1、你好，我先介绍下自己。我是一名中国民航局的监察员。

2、This is my inspector certificate/ID, and we’re doing a ramp inspection today.

2、这是我的监察员证。我们今天的任务是进行一次停机坪检查。

3、I am the operation inspector, he/she is the airworthiness inspector, and we will do the inspection separately.

3、我是运行监察员，他/她是适航监察员，我们将分别检查运行和适航部分。

4、Good morning, Captain. We’re CAAC inspectors and we’re doing a ramp inspection today. Hope you can cooperate with us. Let me introduce my team to you. I’m XXX. This is my inspector certificate/ID…

4、上午好，机长！我们是只能过民航局的监察员，我们今天要进行停机坪检查，希望你配合我们的工作。让我来介绍一下我们团队，我是XXX，这是我的监察员证…

六、结束篇

1、There is no finding today, please sign here.

1、今天的检查没有发现问题，请在这里签名。

2、These are all the findings. Please go through them and sign here if you have no questions. Please take the worksheet to your headquarters.

2、这是今天发现的所有问题，请核实。如果没有异议的话，请在这里签名，并带回总部。

3、That’s all for today. Thanks for your cooperation. Have a good journey!

3、今天的检查到此结束，谢谢你们的配合，旅途愉快！

- 外航检查情景对话

1、情景对话：Scene I：Cockpit驾驶舱

- Inspector: Good morning, Captain. We’re CAAC inspectors and we’re doing a ramp inspection today. Hope you can cooperate with us.

- Captain: Nice to meet you.

- Inspector: Let me introduce my team to you. I’m XXX, operation inspector. This is my inspector certificate. And my colleague is an airworthiness inspector (showing his inspector certificate), and we will do the inspection separately today.

- Captain: OK. Got it. We will cooperate with you as possible as we can.

- Inspector: Sir, could you show us the license and Medical Certificate？And also the ICAO English Proficiency and the expiration date.

- Captain: OK. Wait a minute. This is mine. Bill, where is yours?

- Bill: Here you are.

- Inspector: Thanks. Since there is limitation in the Medical Certificate that you should equip the spare correcting lenses, could you show it?

- Bill: Yes. No problem. They are in my bags. I’ll take them to you.

2、情景对话：Scene II： Cabin 客舱

- Inspector: Hello, I’m cabin crew inspector. I will do routine inspection today. Who is the chief attendant?

- Purser: I am.

- Inspector: how many attendants are there in this flight?

- Purser: Six, including me. I’d like to call all the attendants to come here first.

- Inspector: OK. Please show me your IDs. (checking) Oh, you are Chinese. So you also need to show me your medical certificate.

- Cabin crew: OK. This is my medical certificate. Here you are.

- Inspector: Great. Thanks.

Then checking the row…

- Inspector: Excuse me, there is no passenger briefing card in 11D.

- Cabin crew: I am sorry about that. It’s our fault.